UNIVERSITY OF ARCHITECTURE, CIVIL ENGINEERING AND GEODESY

Department of Reinforced Concrete Structures

COURSE PROJECT IN REINFORCED CONCRETE

Explanatory note with static analysis and design calculations

Reinforced concrete structure of the slab above the first floor of an administrative building

| Student | Dionisios Papadimitriou |
| --- | --- |
| Faculty number | 17945 |
| Date | 07.10.2026 |

Table 1. Input data (assignment)

| Input data | Value |
| --- | --- |
| a [cm] | 630 |
| b [cm] | 580 |
| c [cm] | 350 |
| d [cm] | 170 |
| h_s [cm] | 310 |
| h_b [cm] | 290 |
| q_k1 [kN/m²] | 5.6 |
| q_k2 [kN/m²] | 3.3 |
| R_0 [kN/m²] | 230 |

Contents

Right-click here and choose "Update field" to build the table of contents.

# General data

Codes: EN 1990, EN 1991-1-1, EN 1992-1-1 (Eurocode 2) with the recommended values. Units: kN, m (statics); N, mm (design of sections).

Materials: concrete C25/30, reinforcing steel B500. Partial factors: γ_G = 1.35, γ_Q = 1.5, γ_c = 1.5, γ_s = 1.15.

Table 2. Structural grid (from the formwork plan)

| Axis | x [cm] |  | Axis | y [cm] |
| --- | --- | --- | --- | --- |
| 1 | 0 |  | A (А) | 0 |
| 2 | 630 |  | B (Б) | 580 |
| 3 | 1260 |  | C (В) | 755 |
| 4 | 1645 |  | D (Г) | 930 |
| 5' | 2070 |  | E (Д) | 1510 |
| 5 | 2275 |  |  |  |
| 6 | 2905 |  |  |  |
| 7 | 3535 |  |  |  |

Exhibition hall: 6 one-way slabs per bay, secondary beams every 251.7 cm (l/l = 2.503 ≥ 2). Meeting room: 6 one-way slabs, secondary beams every 244.2 cm (l/l = 2.375 ≥ 2).

Storey heights: first storey h_b = 290 cm, upper storeys h_s = 310 cm (4 storeys). Level of the designed slab: +2.9 m.

# I. Design of reinforced concrete single-span simply supported slab

Slab 17 - corridor between axes B and D (Б-Г) and axes 4-7. The slab is supported on the beams on axes B and D and spans in the short direction.

## 1. Static scheme

Clear span in the long direction

lm​ax,cl=1865 cm

Clear span in the short direction

lm​in,cl=325 cm

Ratio of the spans

λ=lm​ax,cllm​in,cl=1865325=5.738

λ=5.738 > 2

✔ Slab spanning in one direction (simply supported strip)

Thickness of the slab

hf=14 cm

EC2 5.3.2.2(1): a_i = min(h/2; t/2)

a1=minhf2t2=min142252=7 cm

Effective span l_eff = l_n + a_1 + a_2

leff=lm​in,cl+2·a1=325+2·7=339 cm

## 2. Actions

Characteristic values - permanent actions:

Self-weight of the slab (γ = 25 kN/m³)

gk,1=hf·25=0.14·25=3.5 kN/m²

Floor finishes, 5 cm (γ = 22 kN/m³)

gk,2=0.05·22=1.1 kN/m²

Ceiling plaster, 2 cm (γ = 18 kN/m³)

gk,3=0.02·18=0.36 kN/m²

Total permanent action

Σgk=gk,1+gk,2+gk,3=3.5+1.1+0.36=4.96 kN/m²

Characteristic value - variable action:

Imposed load - corridor (category B/C)

qk=3.5 kN/m²

Design values (EN 1990, 6.10):

Permanent action, γ_G = 1.35

gd=1.35·Σgk=1.35·4.96=6.696 kN/m²

Variable action, γ_Q = 1.5

qd=1.5·qk=1.5·3.5=5.25 kN/m²

Total design load

pd=gd+qd=6.696+5.25=11.95 kN/m²

Figure 1. Static scheme of the simply supported slab

## 3. Static analysis

Maximum bending moment (mid-span)

MEd=pd·leff28=11.95·3.3928=17.16 kNm/m

Maximum shear force (supports)

VEd=pd·leff2=11.95·3.392=20.25 kN/m

Figure 2. Internal forces in the simply supported slab

## 4. Design of the bending reinforcement

Design characteristics of the materials (EN 1992-1-1):

Concrete C25/30, reinforcing steel B500.

Characteristic cylinder strength of concrete

fck=25 N/mm²

Design compressive strength

fcd=αcc·fckγc=1·251.5=16.67 N/mm²

Mean tensile strength

fctm=2.56 N/mm²

Characteristic yield strength of steel

fyk=500 N/mm²

Design yield strength of steel

fyd=fykγs=5001.15=434.8 N/mm²

Effective depth of the cross-section:

c_nom = 25 mm (XC1, 50 years), assumed bar Φ10

d=hf−cnom−φ2=140−25−102=110 mm

Mid-span section (b = 1000 mm, per metre)

Relative design moment

μEd=MEd·10⁶b·d2·fcd=17.16·10⁶1000·1102·16.67=0.0851

μEd=0.0851 ≤ μl​im=0.295

✔ Single reinforcement is sufficient

Lever arm ratio z/d

ζ=0.5·1+1−2·μEd=0.5·1+1−2·0.0851=0.9555

Required reinforcement

As,req=MEd·10⁶ζ·d·fyd=17.16·10⁶0.9555·110·434.8=375.5 mm²/m

Minimum reinforcement (EC2 9.2.1.1)

As,m​in=max0.26·fctmfyk·b·d0.0013·b·d

=max0.26·2.56500·1000·1100.0013·1000·110=146.7 mm²/m

Provided reinforcement: 5Φ10/20 cm  →  A_s,prov = 392.7 mm²/m

Required reinforcement ratio

ρ=Asb·d=375.51000·110=0.00341

Provided reinforcement ratio

ρprov=As,provb·d=392.71000·110=0.00357

## 5. Design for shear

Size factor

k=min1+200d2=min1+2001102=2

vm​in=0.035·k32·fck=0.035·232·25=0.495 N/mm²

Shear resistance without shear reinforcement (EC2 6.2.2)

VRd,c=CRd,c·k·3100·ρl·fck·b·d10³=0.12·2·3100·0.00357·25·1000·11010³=54.76 kN/m

VRd,c,m​in=vm​in·b·d10³=0.495·1000·11010³=54.45 kN/m

V_Rd,c = max(V_Rd,c ; V_Rd,c,min) = 54.76 kN/m

VEd=20.25 kN/m ≤ VRd,c=54.76 kN/m

✔ No shear reinforcement is required

## 6. Control of deflection (EC2 7.4.2)

Simply supported slab: K = 1.0

ρ0=fck·10⁻³=25·10⁻³=0.005

Basic span/depth ratio, ρ ≤ ρ_0 (EC2 7.16a)

(l/d)basic=K·11+1.5·fck·ρ0ρ+3.2·fck·ρ0ρ−132

=1·11+1.5·25·0.0050.00341+3.2·25·0.0050.00341−132=27.05

Steel stress under the quasi-permanent load

σs=fyd·gk+0.3·qkgd+qd·As,reqAs,prov=434.8·4.96+0.3·3.56.696+5.25·375.5392.7=209.2 N/mm²

Correction factor 310/σ_s (≤ 1.5)

kσ=min310σs1.5=min310209.21.5=1.482

(l/d)l​im=(l/d)basic·kσ=27.05·1.482=40.09

l/d=leffd=3390110=30.82

l/d=30.82 ≤ (l/d)l​im=40.09

✔ Deflection need not be calculated (l/d check satisfied)

## 7. Detailing

Detailing of the reinforcement (EC2 9.3.1):

Distribution (secondary) reinforcement: A_s,dist ≥ max(0.20·A_s; 3Φ8/m) = 150.8 mm²/m → Φ6/17.5 cm

Spacing of main bars ≤ min(2·h_f; 250 mm) = 250 mm (adopted 200 mm); distribution bars ≤ min(3·h_f; 400 mm).

Top reinforcement at end supports (monolithic connection): A_s3 ≥ max(0.25·A_s,span; A_s,min; 5Φ8/m), length 0.2·l_min from the face of the support + l_bd.

Bottom bars are anchored at end supports with l_bd and lapped over interior supports; top bars over interior supports extend 0.3·l_min beyond the face of the support.

# II. Design of reinforced concrete continuous slab spanning in one direction - elastic analysis

Slabs 1-6 - exhibition hall, axes 1-3 / A-E. The slab is continuous over the secondary beams B5... (5 interior supports at a spacing of 251.7 cm) and spans from axis A to axis E (Д) in 6 spans.

## 1. Static scheme

Clear span in the long direction (wall 25 cm on axis 1, beam 30 cm on axis 2)

lm​ax,cl=602.5 cm

Clear span between the secondary beams (b_w = 25 cm)

lm​in,cl=226.7 cm

λ=lm​ax,cllm​in,cl=602.5226.7=2.658

λ=2.658 > 2

✔ Slab spanning in one direction - continuous strip 1 m wide

Thickness of the slab

hf=12 cm

End span: l_eff,1 = l_n + min(h_f/2; t/2) + t/2

leff,1=lm​in,cl+minhf2t2+t2=226.7+min122252+252=245.2 cm

Interior spans: l_eff,2 = l_n + t

leff,2=lm​in,cl+t=226.7+25=251.7 cm

## 2. Actions

Characteristic values - permanent actions:

Self-weight of the slab (γ = 25 kN/m³)

gk,1=hf·25=0.12·25=3 kN/m²

Floor finishes, 5 cm (γ = 22 kN/m³)

gk,2=0.05·22=1.1 kN/m²

Ceiling plaster, 2 cm (γ = 18 kN/m³)

gk,3=0.02·18=0.36 kN/m²

Total permanent action

Σgk=gk,1+gk,2+gk,3=3+1.1+0.36=4.46 kN/m²

Characteristic value - variable action:

Imposed load - exhibition hall q_k1 (assignment)

qk=5.6 kN/m²

Design values (EN 1990, 6.10):

Permanent action, γ_G = 1.35

gd=1.35·Σgk=1.35·4.46=6.021 kN/m²

Variable action, γ_Q = 1.5

qd=1.5·qk=1.5·5.6=8.4 kN/m²

Total design load

pd=gd+qd=6.021+8.4=14.42 kN/m²

## 3. Static analysis - bending moments and shear forces

The strip is analysed as a continuous beam with 6 spans (stiffness method). The permanent load acts on all spans; the variable load is placed span by span in the most unfavourable arrangement for every section (pattern loading). The envelope is obtained by superposition, the results are written in the familiar coefficient form M = (α_g·g_d + α_q·q_d)·l².

Figure 3. Static scheme of the continuous slab

Span 1 (x = 1.042 m)

maxMEd,1=αg·gd+αq·qd·leff,12=0.0765·6.021+0.1005·8.4·2.4522=7.84 kNm/m

Span 2 (x = 3.757 m)

maxMEd,2=αg·gd+αq·qd·leff,22=0.0352·6.021+0.0791·8.4·2.5172=5.554 kNm/m

Span 3 (x = 6.211 m)

maxMEd,3=αg·gd+αq·qd·leff,22=0.043·6.021+0.0837·8.4·2.5172=6.094 kNm/m

Support B

minMB=−αg·gd+αq·qd·leff2=−0.1021·6.021+0.1164·8.4·2.5172=-10.09 kNm/m

Support C

minMC=−αg·gd+αq·qd·leff2=−0.078·6.021+0.1101·8.4·2.5172=-8.832 kNm/m

Support D

minMD=−αg·gd+αq·qd·leff2=−0.086·6.021+0.1148·8.4·2.5172=-9.386 kNm/m

Maximum shear force (left of support B)

maxVEd=βg·gd+βq·qd·leff,1=0.6076·6.021+0.6227·8.4·2.452=21.79 kN/m

Table 3. Coefficients and design values from the elastic analysis

| Section | α_g (β_g) | α_q (β_q) | Design value [kNm/m, kN/m] |
| --- | --- | --- | --- |
| Span 1 | 0.0765 | 0.1005 | 7.84 |
| Span 2 | 0.0352 | 0.0791 | 5.554 |
| Span 3 | 0.043 | 0.0837 | 6.094 |
| Support B | -0.1021 | -0.1164 | -10.09 |
| Support C | -0.078 | -0.1101 | -8.832 |
| Support D | -0.086 | -0.1148 | -9.386 |
| Shear at B (left) | 0.6076 | 0.6227 | 21.79 |

Figure 4. Envelopes of M and V for the continuous slab (pattern loading)

## 4. Design of the bending reinforcement

Design characteristics of the materials (EN 1992-1-1):

Concrete C25/30, reinforcing steel B500.

Characteristic cylinder strength of concrete

fck=25 N/mm²

Design compressive strength

fcd=αcc·fckγc=1·251.5=16.67 N/mm²

Mean tensile strength

fctm=2.56 N/mm²

Characteristic yield strength of steel

fyk=500 N/mm²

Design yield strength of steel

fyd=fykγs=5001.15=434.8 N/mm²

Effective depth of the cross-section:

c_nom = 25 mm (XC1, 50 years), assumed bar Φ10

d=hf−cnom−φ2=120−25−102=90 mm

Span 1 (bottom reinforcement)

Relative design moment

μEd,1=maxMEd,1·10⁶b·d2·fcd=7.84·10⁶1000·902·16.67=0.0581

μEd,1=0.0581 ≤ μl​im=0.295

✔ Single reinforcement is sufficient

Lever arm ratio z/d

ζ1=0.5·1+1−2·μEd,1=0.5·1+1−2·0.0581=0.9701

Required reinforcement

As,req,1=maxMEd,1·10⁶ζ1·d·fyd=7.84·10⁶0.9701·90·434.8=206.5 mm²/m

Minimum reinforcement (EC2 9.2.1.1)

As,m​in,1=max0.26·fctmfyk·b·d0.0013·b·d=max0.26·2.56500·1000·900.0013·1000·90=120 mm²/m

Provided reinforcement: 5Φ8/20 cm  →  A_s,prov = 251.3 mm²/m

Required reinforcement ratio

ρ1=Asb·d=206.51000·90=0.00229

Provided reinforcement ratio

ρprov,1=As,provb·d=251.31000·90=0.00279

Span 2 (bottom reinforcement)

Relative design moment

μEd,2=maxMEd,2·10⁶b·d2·fcd=5.554·10⁶1000·902·16.67=0.0411

μEd,2=0.0411 ≤ μl​im=0.295

✔ Single reinforcement is sufficient

Lever arm ratio z/d

ζ2=0.5·1+1−2·μEd,2=0.5·1+1−2·0.0411=0.979

Required reinforcement

As,req,2=maxMEd,2·10⁶ζ2·d·fyd=5.554·10⁶0.979·90·434.8=145 mm²/m

Minimum reinforcement (EC2 9.2.1.1)

As,m​in,2=max0.26·fctmfyk·b·d0.0013·b·d=max0.26·2.56500·1000·900.0013·1000·90=120 mm²/m

Provided reinforcement: 5Φ8/20 cm  →  A_s,prov = 251.3 mm²/m

Required reinforcement ratio

ρ2=Asb·d=1451000·90=0.00161

Provided reinforcement ratio

ρprov,2=As,provb·d=251.31000·90=0.00279

Span 3 (bottom reinforcement)

Relative design moment

μEd,3=maxMEd,3·10⁶b·d2·fcd=6.094·10⁶1000·902·16.67=0.0451

μEd,3=0.0451 ≤ μl​im=0.295

✔ Single reinforcement is sufficient

Lever arm ratio z/d

ζ3=0.5·1+1−2·μEd,3=0.5·1+1−2·0.0451=0.9769

Required reinforcement

As,req,3=maxMEd,3·10⁶ζ3·d·fyd=6.094·10⁶0.9769·90·434.8=159.4 mm²/m

Minimum reinforcement (EC2 9.2.1.1)

As,m​in,3=max0.26·fctmfyk·b·d0.0013·b·d=max0.26·2.56500·1000·900.0013·1000·90=120 mm²/m

Provided reinforcement: 5Φ8/20 cm  →  A_s,prov = 251.3 mm²/m

Required reinforcement ratio

ρ3=Asb·d=159.41000·90=0.00177

Provided reinforcement ratio

ρprov,3=As,provb·d=251.31000·90=0.00279

Support B (top reinforcement) - moment at the face of the support:

ΔM = V_Ed·t/2

ΔMEd=VEd,B·t2=21.79·0.252=2.724 kNm/m

Not less than 65 % of the fixed-end moment (EC2 5.3.2.2(4))

MEd,B=maxminMB−ΔMEd0.65·p·l²/12=max-10.09−2.7244.947=7.363 kNm/m

Relative design moment

μEd,B=MEd,B·10⁶b·d2·fcd=7.363·10⁶1000·902·16.67=0.0545

μEd,B=0.0545 ≤ μl​im=0.295

✔ Single reinforcement is sufficient

Lever arm ratio z/d

ζB=0.5·1+1−2·μEd,B=0.5·1+1−2·0.0545=0.9719

Required reinforcement

As,req,B=MEd,B·10⁶ζB·d·fyd=7.363·10⁶0.9719·90·434.8=193.6 mm²/m

Minimum reinforcement (EC2 9.2.1.1)

As,m​in,B=max0.26·fctmfyk·b·d0.0013·b·d=max0.26·2.56500·1000·900.0013·1000·90=120 mm²/m

Provided reinforcement: 5Φ8/20 cm  →  A_s,prov = 251.3 mm²/m

Required reinforcement ratio

ρB=Asb·d=193.61000·90=0.00215

Provided reinforcement ratio

ρprov,B=As,provb·d=251.31000·90=0.00279

Support C (top reinforcement) - moment at the face of the support:

ΔM = V_Ed·t/2

ΔMEd=VEd,C·t2=19.84·0.252=2.48 kNm/m

Not less than 65 % of the fixed-end moment (EC2 5.3.2.2(4))

MEd,C=maxminMC−ΔMEd0.65·p·l²/12=max-8.832−2.484.947=6.351 kNm/m

Relative design moment

μEd,C=MEd,C·10⁶b·d2·fcd=6.351·10⁶1000·902·16.67=0.047

μEd,C=0.047 ≤ μl​im=0.295

✔ Single reinforcement is sufficient

Lever arm ratio z/d

ζC=0.5·1+1−2·μEd,C=0.5·1+1−2·0.047=0.9759

Required reinforcement

As,req,C=MEd,C·10⁶ζC·d·fyd=6.351·10⁶0.9759·90·434.8=166.3 mm²/m

Minimum reinforcement (EC2 9.2.1.1)

As,m​in,C=max0.26·fctmfyk·b·d0.0013·b·d=max0.26·2.56500·1000·900.0013·1000·90=120 mm²/m

Provided reinforcement: 5Φ8/20 cm  →  A_s,prov = 251.3 mm²/m

Required reinforcement ratio

ρC=Asb·d=166.31000·90=0.00185

Provided reinforcement ratio

ρprov,C=As,provb·d=251.31000·90=0.00279

Support D (top reinforcement) - moment at the face of the support:

ΔM = V_Ed·t/2

ΔMEd=VEd,D·t2=20.26·0.252=2.533 kNm/m

Not less than 65 % of the fixed-end moment (EC2 5.3.2.2(4))

MEd,D=maxminMD−ΔMEd0.65·p·l²/12=max-9.386−2.5334.947=6.853 kNm/m

Relative design moment

μEd,D=MEd,D·10⁶b·d2·fcd=6.853·10⁶1000·902·16.67=0.0508

μEd,D=0.0508 ≤ μl​im=0.295

✔ Single reinforcement is sufficient

Lever arm ratio z/d

ζD=0.5·1+1−2·μEd,D=0.5·1+1−2·0.0508=0.9739

Required reinforcement

As,req,D=MEd,D·10⁶ζD·d·fyd=6.853·10⁶0.9739·90·434.8=179.8 mm²/m

Minimum reinforcement (EC2 9.2.1.1)

As,m​in,D=max0.26·fctmfyk·b·d0.0013·b·d=max0.26·2.56500·1000·900.0013·1000·90=120 mm²/m

Provided reinforcement: 5Φ8/20 cm  →  A_s,prov = 251.3 mm²/m

Required reinforcement ratio

ρD=Asb·d=179.81000·90=0.002

Provided reinforcement ratio

ρprov,D=As,provb·d=251.31000·90=0.00279

## 5. Design for shear

Size factor

k=min1+200d2=min1+200902=2

vm​in=0.035·k32·fck=0.035·232·25=0.495 N/mm²

Shear resistance without shear reinforcement (EC2 6.2.2)

VRd,c=CRd,c·k·3100·ρl·fck·b·d10³=0.12·2·3100·0.00279·25·1000·9010³=41.28 kN/m

VRd,c,m​in=vm​in·b·d10³=0.495·1000·9010³=44.55 kN/m

V_Rd,c = max(V_Rd,c ; V_Rd,c,min) = 44.55 kN/m

maxVEd=21.79 kN/m ≤ VRd,c=44.55 kN/m

✔ No shear reinforcement is required

## 6. Control of deflection (EC2 7.4.2)

End span of a continuous slab: K = 1.3

ρ0=fck·10⁻³=25·10⁻³=0.005

Basic span/depth ratio, ρ ≤ ρ_0 (EC2 7.16a)

(l/d)basic=K·11+1.5·fck·ρ0ρ+3.2·fck·ρ0ρ−132

=1.3·11+1.5·25·0.0050.00229+3.2·25·0.0050.00229−132=62.17

Steel stress under the quasi-permanent load

σs=fyd·gk+0.3·qkgd+qd·As,reqAs,prov=434.8·4.46+0.3·5.66.021+8.4·206.5251.3=152.1 N/mm²

Correction factor 310/σ_s (≤ 1.5)

kσ=min310σs1.5=min310152.11.5=1.5

(l/d)l​im=(l/d)basic·kσ=62.17·1.5=93.25

l/d=leffd=245290=27.24

l/d=27.24 ≤ (l/d)l​im=93.25

✔ Deflection need not be calculated (l/d check satisfied)

## 7. Detailing

Detailing of the reinforcement (EC2 9.3.1):

Distribution (secondary) reinforcement: A_s,dist ≥ max(0.20·A_s; 3Φ8/m) = 150.8 mm²/m → Φ6/17.5 cm

Spacing of main bars ≤ min(2·h_f; 250 mm) = 240 mm (adopted 200 mm); distribution bars ≤ min(3·h_f; 400 mm).

Top reinforcement at end supports (monolithic connection): A_s3 ≥ max(0.25·A_s,span; A_s,min; 5Φ8/m), length 0.2·l_min from the face of the support + l_bd.

Bottom bars are anchored at end supports with l_bd and lapped over interior supports; top bars over interior supports extend 0.3·l_min beyond the face of the support.

Table 4. Reinforcement of the continuous slab

| Section | M_Ed [kNm/m] | A_s,req [mm²/m] | Adopted reinforcement | A_s,prov [mm²/m] |
| --- | --- | --- | --- | --- |
| Span 1 | 7.84 | 206.5 | 5Φ8/20 cm | 251.3 |
| Span 2 | 5.554 | 145 | 5Φ8/20 cm | 251.3 |
| Span 3 | 6.094 | 159.4 | 5Φ8/20 cm | 251.3 |
| Support B | 10.09 | 193.6 | 5Φ8/20 cm | 251.3 |
| Support C | 8.832 | 166.3 | 5Φ8/20 cm | 251.3 |
| Support D | 9.386 | 179.8 | 5Φ8/20 cm | 251.3 |

## 8. Reinforcement with welded wire meshes

The slabs are reinforced with welded wire meshes of type R (main bars in the long direction of the sheet, standard sheet 5 × 2.15 m), main bars parallel to the span. Bottom meshes are placed in every panel and run into the beams up to the far face minus the cover; top meshes are strips over the beams (0.3·l beyond the face of the beam) and at the edges. Laps of the transverse bars ≥ 150 mm.

Table 5. Welded wire meshes (R-type)

| Mesh | Main bars | Transverse bars | A_s [mm²/m] | A_s,tr [mm²/m] |
| --- | --- | --- | --- | --- |
| R131 | Φ5/15 cm | Φ4.2/25 cm | 130.9 | 55.42 |
| R188 | Φ6/15 cm | Φ4.2/25 cm | 188.5 | 55.42 |
| R221 | Φ6.5/15 cm | Φ4.5/25 cm | 221.2 | 63.62 |
| R257 | Φ7/15 cm | Φ4.5/25 cm | 256.6 | 63.62 |
| R335 | Φ8/15 cm | Φ5/25 cm | 335.1 | 78.54 |
| R424 | Φ9/15 cm | Φ5.5/25 cm | 424.1 | 95.03 |
| R524 | Φ10/15 cm | Φ6/25 cm | 523.6 | 113.1 |

Table 6. Welded wire meshes of the continuous slab

| Section | A_s,req [mm²/m] | Adopted mesh | A_s,prov [mm²/m] |
| --- | --- | --- | --- |
| Span 1 (bottom) | 206.5 | R221 | 221.2 |
| Span 2 (bottom) | 145 | R188 | 188.5 |
| Span 3 (bottom) | 159.4 | R188 | 188.5 |
| Support B (top) | 193.6 | R221 | 221.2 |
| Support C (top) | 166.3 | R188 | 188.5 |
| Support D (top) | 179.8 | R188 | 188.5 |
| End supports A, E (top): max(0.25·A_s; A_s,min; 5Φ8/m) | 251.3 | R257 | 256.6 |
| Edges on axes 1, 2, 3 (top, constructive 5Φ8/m) | 251.3 | R257 | 256.6 |

Control of deflection with the mesh of the end span (R221):

ρ0=fck·10⁻³=25·10⁻³=0.005

Basic span/depth ratio, ρ ≤ ρ_0 (EC2 7.16a)

(l/d)basic=K·11+1.5·fck·ρ0ρ+3.2·fck·ρ0ρ−132

=1.3·11+1.5·25·0.0050.00229+3.2·25·0.0050.00229−132=62.17

Steel stress under the quasi-permanent load

σs=fyd·gk+0.3·qkgd+qd·As,reqAs,prov=434.8·4.46+0.3·5.66.021+8.4·206.5221.2=172.8 N/mm²

Correction factor 310/σ_s (≤ 1.5)

kσ=min310σs1.5=min310172.81.5=1.5

(l/d)l​im=(l/d)basic·kσ=62.17·1.5=93.25

l/d=leffd=245290=27.24

l/d=27.24 ≤ (l/d)l​im=93.25

✔ Deflection need not be calculated (l/d check satisfied)

Spacing of the main bars of the meshes 150 mm ≤ min(2·h_f; 250 mm) = 240 mm.

# III. Design of reinforced concrete continuous slab spanning in one direction - plastic analysis

Slabs 11, 12, 13, 14, 15, 16 - meeting room, axes 5'-7 / D-E. The slab is continuous over 5 secondary beams (spacing 244.2 cm) between the walls on axes 5' and 7.

## 1. Static scheme

Clear span in the long direction (axes D-E)

lm​ax,cl=555 cm

Clear span between the secondary beams

lm​in,cl=219.2 cm

λ=lm​ax,cllm​in,cl=555219.2=2.532

λ=2.532 > 2

✔ Slab spanning in one direction

Thickness of the slab

hf=12 cm

End spans

leff,1=lm​in,cl+minhf2t2+t2=219.2+min122252+252=237.7 cm

Interior spans

leff,2=lm​in,cl+t=219.2+25=244.2 cm

## 2. Actions

Characteristic values - permanent actions:

Self-weight of the slab (γ = 25 kN/m³)

gk,1=hf·25=0.12·25=3 kN/m²

Floor finishes, 5 cm (γ = 22 kN/m³)

gk,2=0.05·22=1.1 kN/m²

Ceiling plaster, 2 cm (γ = 18 kN/m³)

gk,3=0.02·18=0.36 kN/m²

Total permanent action

Σgk=gk,1+gk,2+gk,3=3+1.1+0.36=4.46 kN/m²

Characteristic value - variable action:

Imposed load - meeting room q_k2 (assignment)

qk=3.3 kN/m²

Design values (EN 1990, 6.10):

Permanent action, γ_G = 1.35

gd=1.35·Σgk=1.35·4.46=6.021 kN/m²

Variable action, γ_Q = 1.5

qd=1.5·qk=1.5·3.3=4.95 kN/m²

Total design load

pd=gd+qd=6.021+4.95=10.97 kN/m²

## 3. Static analysis - plastic analysis (moment redistribution)

For slabs with approximately equal spans (difference ≤ 20 %) and q_k ≤ 1.25 g_k... the design effects may be taken with the coefficients of the plastic analysis (course table):

Table 7. Coefficients of the plastic analysis

| Section | Moment | Shear |
| --- | --- | --- |
| End span | p·l²/11 | 0.40·p·l (outer support) |
| First interior support | −p·l²/11 | 0.60·p·l |
| Interior spans and supports | ± p·l²/16 | 0.50·p·l |

End span

MEd,1=pd·leff,1211=10.97·2.377211=5.634 kNm/m

First interior support (absolute value)

MEd,B=pd·maxleff,1leff,2211=10.97·max2.3772.442211=5.946 kNm/m

Interior spans and interior supports

MEd,2=pd·leff,2216=10.97·2.442216=4.088 kNm/m

Outer support

VEd,1=0.4·pd·leff,1=0.4·10.97·2.377=10.43 kN/m

First interior support

VEd,2=0.6·pd·leff,1=0.6·10.97·2.377=15.64 kN/m

Interior supports

VEd,3=0.5·pd·leff,2=0.5·10.97·2.442=13.39 kN/m

Figure 5. Static scheme of the slabs in the meeting room

Figure 6. Bending moments and shear forces - plastic analysis

## 4. Design of the bending reinforcement

Design characteristics of the materials (EN 1992-1-1):

Concrete C25/30, reinforcing steel B500.

Characteristic cylinder strength of concrete

fck=25 N/mm²

Design compressive strength

fcd=αcc·fckγc=1·251.5=16.67 N/mm²

Mean tensile strength

fctm=2.56 N/mm²

Characteristic yield strength of steel

fyk=500 N/mm²

Design yield strength of steel

fyd=fykγs=5001.15=434.8 N/mm²

Effective depth of the cross-section:

c_nom = 25 mm (XC1, 50 years), assumed bar Φ10

d=hf−cnom−φ2=120−25−102=90 mm

End span (bottom)

Relative design moment

μEd,1=MEd,1·10⁶b·d2·fcd=5.634·10⁶1000·902·16.67=0.0417

μEd,1=0.0417 ≤ μl​im=0.295

✔ Single reinforcement is sufficient

Lever arm ratio z/d

ζ1=0.5·1+1−2·μEd,1=0.5·1+1−2·0.0417=0.9787

Required reinforcement

As,req,1=MEd,1·10⁶ζ1·d·fyd=5.634·10⁶0.9787·90·434.8=147.1 mm²/m

Minimum reinforcement (EC2 9.2.1.1)

As,m​in,1=max0.26·fctmfyk·b·d0.0013·b·d=max0.26·2.56500·1000·900.0013·1000·90=120 mm²/m

Provided reinforcement: 5Φ8/20 cm  →  A_s,prov = 251.3 mm²/m

Required reinforcement ratio

ρ1=Asb·d=147.11000·90=0.00163

Provided reinforcement ratio

ρprov,1=As,provb·d=251.31000·90=0.00279

First interior support (top)

Relative design moment

μEd,B=MEd,B·10⁶b·d2·fcd=5.946·10⁶1000·902·16.67=0.044

μEd,B=0.044 ≤ μl​im=0.295

✔ Single reinforcement is sufficient

Lever arm ratio z/d

ζB=0.5·1+1−2·μEd,B=0.5·1+1−2·0.044=0.9775

Required reinforcement

As,req,B=MEd,B·10⁶ζB·d·fyd=5.946·10⁶0.9775·90·434.8=155.5 mm²/m

Minimum reinforcement (EC2 9.2.1.1)

As,m​in,B=max0.26·fctmfyk·b·d0.0013·b·d=max0.26·2.56500·1000·900.0013·1000·90=120 mm²/m

Provided reinforcement: 5Φ8/20 cm  →  A_s,prov = 251.3 mm²/m

Required reinforcement ratio

ρB=Asb·d=155.51000·90=0.00173

Provided reinforcement ratio

ρprov,B=As,provb·d=251.31000·90=0.00279

Interior spans (bottom) and interior supports (top)

Relative design moment

μEd,2=MEd,2·10⁶b·d2·fcd=4.088·10⁶1000·902·16.67=0.0303

μEd,2=0.0303 ≤ μl​im=0.295

✔ Single reinforcement is sufficient

Lever arm ratio z/d

ζ2=0.5·1+1−2·μEd,2=0.5·1+1−2·0.0303=0.9846

Required reinforcement

As,req,2=MEd,2·10⁶ζ2·d·fyd=4.088·10⁶0.9846·90·434.8=106.1 mm²/m

Minimum reinforcement (EC2 9.2.1.1)

As,m​in,2=max0.26·fctmfyk·b·d0.0013·b·d=max0.26·2.56500·1000·900.0013·1000·90=120 mm²/m

Provided reinforcement: 5Φ8/20 cm  →  A_s,prov = 251.3 mm²/m

Required reinforcement ratio

ρ2=Asb·d=1201000·90=0.00133

Provided reinforcement ratio

ρprov,2=As,provb·d=251.31000·90=0.00279

Ductility requirement for plastic analysis (EC2 5.6.2(2)): x_u/d ≤ 0.25 (C ≤ 50/60)

xu/d,1=1−ζ0.4=1−0.97870.4=0.0533

xu/d,1=0.0533 ≤ 0.25

✔ Rotation capacity is sufficient

xu/d,B=1−ζ0.4=1−0.97750.4=0.0563

xu/d,B=0.0563 ≤ 0.25

✔ Rotation capacity is sufficient

xu/d,2=1−ζ0.4=1−0.98460.4=0.0384

xu/d,2=0.0384 ≤ 0.25

✔ Rotation capacity is sufficient

## 5. Design for shear

V_Ed,max = max(V_Ed,1; V_Ed,2; V_Ed,3) = 15.64 kN/m

Size factor

k=min1+200d2=min1+200902=2

vm​in=0.035·k32·fck=0.035·232·25=0.495 N/mm²

Shear resistance without shear reinforcement (EC2 6.2.2)

VRd,c=CRd,c·k·3100·ρl·fck·b·d10³=0.12·2·3100·0.00279·25·1000·9010³=41.28 kN/m

VRd,c,m​in=vm​in·b·d10³=0.495·1000·9010³=44.55 kN/m

V_Rd,c = max(V_Rd,c ; V_Rd,c,min) = 44.55 kN/m

VEd,m​ax=15.64 kN/m ≤ VRd,c=44.55 kN/m

✔ No shear reinforcement is required

## 6. Control of deflection (EC2 7.4.2)

End span of a continuous slab: K = 1.3

ρ0=fck·10⁻³=25·10⁻³=0.005

Basic span/depth ratio, ρ ≤ ρ_0 (EC2 7.16a)

(l/d)basic=K·11+1.5·fck·ρ0ρ+3.2·fck·ρ0ρ−132

=1.3·11+1.5·25·0.0050.00163+3.2·25·0.0050.00163−132=105.6

Steel stress under the quasi-permanent load

σs=fyd·gk+0.3·qkgd+qd·As,reqAs,prov=434.8·4.46+0.3·3.36.021+4.95·147.1251.3=126.4 N/mm²

Correction factor 310/σ_s (≤ 1.5)

kσ=min310σs1.5=min310126.41.5=1.5

(l/d)l​im=(l/d)basic·kσ=105.6·1.5=158.4

l/d=leffd=237790=26.41

l/d=26.41 ≤ (l/d)l​im=158.4

✔ Deflection need not be calculated (l/d check satisfied)

## 7. Detailing

Detailing of the reinforcement (EC2 9.3.1):

Distribution (secondary) reinforcement: A_s,dist ≥ max(0.20·A_s; 3Φ8/m) = 150.8 mm²/m → Φ6/17.5 cm

Spacing of main bars ≤ min(2·h_f; 250 mm) = 240 mm (adopted 200 mm); distribution bars ≤ min(3·h_f; 400 mm).

Top reinforcement at end supports (monolithic connection): A_s3 ≥ max(0.25·A_s,span; A_s,min; 5Φ8/m), length 0.2·l_min from the face of the support + l_bd.

Bottom bars are anchored at end supports with l_bd and lapped over interior supports; top bars over interior supports extend 0.3·l_min beyond the face of the support.

Table 8. Reinforcement of the slabs in the meeting room

| Section | M_Ed [kNm/m] | A_s,req [mm²/m] | Adopted | A_s,prov [mm²/m] |
| --- | --- | --- | --- | --- |
| End span | 5.634 | 147.1 | 5Φ8/20 cm | 251.3 |
| First interior support | 5.946 | 155.5 | 5Φ8/20 cm | 251.3 |
| Interior span/support | 4.088 | 120 | 5Φ8/20 cm | 251.3 |

# IV. Design of reinforced concrete slabs spanning in two directions

Slabs 19, 20, 21 - offices, axes 4-7 / A-B. In direction x (along axis A) the slabs are continuous over the beams on axes 5 and 6; in direction y they are supported on the beams on axes A and B. Slabs 22 and 23 are the balconies (cantilevers, l = d) in front of slabs 19 and 21.

## 1. Static scheme

Clear span in direction x (axes 4-5, 5-6, 6-7)

lx,cl=605 cm

Clear span in direction y (axes A-B)

ly,cl=555 cm

Thickness of the slabs

hf=15 cm

Support at a wall / edge beam: min(h/2; t/2)

as=minhf2t2=min152252=7.5 cm

Slabs 19 and 21 (one continuous edge)

leff,x1=lx,cl+as+t2=605+7.5+252=625 cm

Slab 20 (both edges continuous)

leff,x2=lx,cl+t=605+25=630 cm

Direction y

leff,y=ly,cl+2·as=555+2·7.5=570 cm

λ1=leff,yleff,x1=570625=0.912

λ1=0.912 ≥ 0.5

✔ l_max/l_min ≤ 2 - slabs spanning in two directions

Clear length of the balconies (= d)

lc,cl=170 cm

Effective length of the cantilever

lc=lc,cl+t2=170+252=182.5 cm

Figure 7. Slabs in two directions and boundary conditions

## 2. Actions

Characteristic values - permanent actions:

Self-weight of the slab (γ = 25 kN/m³)

gk,1=hf·25=0.15·25=3.75 kN/m²

Floor finishes, 5 cm (γ = 22 kN/m³)

gk,2=0.05·22=1.1 kN/m²

Ceiling plaster, 2 cm (γ = 18 kN/m³)

gk,3=0.02·18=0.36 kN/m²

Total permanent action

Σgk=gk,1+gk,2+gk,3=3.75+1.1+0.36=5.21 kN/m²

Characteristic value - variable action:

Imposed load - offices

qk=3.5 kN/m²

Design values (EN 1990, 6.10):

Permanent action, γ_G = 1.35

gd=1.35·Σgk=1.35·5.21=7.034 kN/m²

Variable action, γ_Q = 1.5

qd=1.5·qk=1.5·3.5=5.25 kN/m²

Total design load

pd=gd+qd=7.034+5.25=12.28 kN/m²

Balconies - characteristic permanent actions:

Self-weight, variable thickness h_f → 12 cm (mean)

gk,c1=hf+htip200·25=15+12200·25=3.375 kN/m²

Perlite concrete 10 cm, screed 4 cm, mosaic 2 cm, plaster 2 cm

gk,c2=0.1·6+0.04·22+0.02·22+0.02·18=2.28 kN/m²

Σgk,c=gk,c1+gk,c2=3.375+2.28=5.655 kN/m²

Imposed load on the balcony

qk,c=3 kN/m²

gd,c=1.35·Σgk,c=1.35·5.655=7.634 kN/m²

qd,c=1.5·qk,c=1.5·3=4.5 kN/m²

Parapet (RC base 15/30 cm and metal railing):

Gk,par=0.15·0.3·25+0.3+0.4+0.15·0.02·18+0.8=2.231 kN/m

Horizontal load on the railing (h = 1.1 m)

Hk=0.5 kN/m

Gd,par=1.35·Gk,par=1.35·2.231=3.012 kN/m

Hd=1.5·Hk=1.5·0.5=0.75 kN/m

## 3. Static analysis

### 3.1 Cantilever slabs (balconies)

Bending moment at the root of the cantilever

MEd,c=gd,c·lc22+qd,c·lc−bpar22+Gd,par·lc−bpar+Hd·1.1

=7.634·1.82522+4.5·1.825−0.07522+3.012·1.825−0.075+0.75·1.1=25.7 kNm/m

VEd,c=gd,c·lc+qd,c·lc−bpar+Gd,par=7.634·1.825+4.5·1.825−0.075+3.012=24.82 kN/m

Characteristic moment from permanent actions (restraint of the slab edge)

Mk,cg=Σgk,c·lc22+Gk,par·lc−bpar=5.655·1.82522+2.231·1.825−0.075=13.32 kNm/m

### 3.2 Verification of the boundary condition between slabs and balconies

First approximation: the edge on axis A is restrained by the cantilever. Load carried in direction y (Marcus distribution, edge y fixed - other edge simply supported):

k = 2 for fixed-simple, k = 5 for simple-simple, k = 1 for fixed-fixed strips

χy=kx·lx4kx·lx4+ky·ly4=2·6.2542·6.254+2·5.74=0.5911

Restraining moment required for a fixed edge (p·l²/8 of the propped strip)

MA,restr=χy·pd·ly28=0.5911·12.28·5.728=29.49 kNm/m

✔ M_A,restr > M_k,cg - the assumption of a fixed edge is NOT confirmed; the edge on axis A is treated as simply supported (case 2 / case 3 tables).

### 3.3 Bending moments in the spans - elastic analysis (plate theory)

The coefficients m are obtained from the theory of elastic plates (Lévy series, ν = 0) - the same basis as the tables of Marcus/Czerny used in the course. M = p·l²/m. For the maximum span moments the load is split into p' = g_d + q_d/2 acting on the slab with its real boundary conditions and p'' = q_d/2 acting on all slabs as simply supported (chequerboard loading).

Table 9. Moment coefficients (plate theory, ν = 0)

| Case | Edges x | l_x [m] | l_y [m] | m_x | m_y | m_x,sup |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | S - S | 6.25 | 5.7 | 33.45 | 22.5 | - |
| 2 | S - C | 6.25 | 5.7 | 34.61 | 31.29 | 13.13 |
| 1 (l_x2) | S - S | 6.3 | 5.7 | 34.09 | 22.16 | - |
| 3 | C - C | 6.3 | 5.7 | 40.03 | 45.33 | 15.42 |

p'=gd+qd2=7.034+5.252=9.659 kN/m²

p''=qd2=5.252=2.625 kN/m²

Slabs 19 and 21 (case 2):

MEd,x=p'mx2+p''mx1·leff,x12=9.65934.61+2.62533.45·6.252=13.97 kNm/m

MEd,y=p'my2+p''my1·leff,y2=9.65931.29+2.62522.5·5.72=13.82 kNm/m

Slab 20 (case 3):

MEd,x,2=p'mx3+p''mx1·leff,x22=9.65940.03+2.62534.09·6.32=12.63 kNm/m

MEd,y,2=p'my3+p''my1·leff,y2=9.65945.33+2.62522.16·5.72=10.77 kNm/m

### 3.4 Bending moments over the supports (axes 5 and 6)

Clamped edge of slab 19

M19=pd·leff,x12mx2,s​up=12.28·6.25213.13=36.53 kNm/m

Clamped edge of slab 20

M20=pd·leff,x22mx3,s​up=12.28·6.3215.42=31.61 kNm/m

Ratio ≥ 0.8 → mean value of the two moments

M19-20=M19+M202=36.53+31.612=34.07 kNm/m

Share of the load carried in direction x (Marcus)

χx=ky·leff,y4kx·leff,x14+ky·leff,y4=5·5.742·6.254+5·5.74=0.6336

Reduction to the face of the beam

ΔM=χx·pd·leff,x12·t2=0.6336·12.28·6.252·0.252=3.04 kNm/m

Design moment over the supports (absolute value)

MEd,56=M19-20−ΔM=34.07−3.04=31.03 kNm/m

### 3.5 Shear forces

At the continuous edge (5/8 of the strip load)

VEd,x=0.625·χx·pd·leff,x1=0.625·0.6336·12.28·6.25=30.4 kN/m

VEd,y=0.5·1−χx·pd·leff,y=0.5·1−0.6336·12.28·5.7=12.83 kN/m

## 4. Design of the bending reinforcement

Design characteristics of the materials (EN 1992-1-1):

Concrete C25/30, reinforcing steel B500.

Characteristic cylinder strength of concrete

fck=25 N/mm²

Design compressive strength

fcd=αcc·fckγc=1·251.5=16.67 N/mm²

Mean tensile strength

fctm=2.56 N/mm²

Characteristic yield strength of steel

fyk=500 N/mm²

Design yield strength of steel

fyd=fykγs=5001.15=434.8 N/mm²

Effective depth of the cross-section:

c_nom = 25 mm (XC1, 50 years), assumed bar Φ10

dx=hf−cnom−φ2=150−25−102=120 mm

Bars in direction y are placed above the bars in x

dy=dx−φ=120−10=110 mm

Slabs 19, 21 - direction x (bottom)

Relative design moment

μEd,x=MEd,x·10⁶b·dx2·fcd=13.97·10⁶1000·1202·16.67=0.0582

μEd,x=0.0582 ≤ μl​im=0.295

✔ Single reinforcement is sufficient

Lever arm ratio z/d

ζx=0.5·1+1−2·μEd,x=0.5·1+1−2·0.0582=0.97

Required reinforcement

As,req,x=MEd,x·10⁶ζx·dx·fyd=13.97·10⁶0.97·120·434.8=276 mm²/m

Minimum reinforcement (EC2 9.2.1.1)

As,m​in,x=max0.26·fctmfyk·b·dx0.0013·b·dx

=max0.26·2.56500·1000·1200.0013·1000·120=160.1 mm²/m

Provided reinforcement: Φ8/17.5 cm  →  A_s,prov = 287.2 mm²/m

Required reinforcement ratio

ρx=Asb·dx=2761000·120=0.0023

Provided reinforcement ratio

ρprov,x=As,provb·dx=287.21000·120=0.00239

Slabs 19, 21 - direction y (bottom)

Relative design moment

μEd,y=MEd,y·10⁶b·dy2·fcd=13.82·10⁶1000·1102·16.67=0.0685

μEd,y=0.0685 ≤ μl​im=0.295

✔ Single reinforcement is sufficient

Lever arm ratio z/d

ζy=0.5·1+1−2·μEd,y=0.5·1+1−2·0.0685=0.9645

Required reinforcement

As,req,y=MEd,y·10⁶ζy·dy·fyd=13.82·10⁶0.9645·110·434.8=299.6 mm²/m

Minimum reinforcement (EC2 9.2.1.1)

As,m​in,y=max0.26·fctmfyk·b·dy0.0013·b·dy

=max0.26·2.56500·1000·1100.0013·1000·110=146.7 mm²/m

Provided reinforcement: Φ8/16 cm  →  A_s,prov = 314.2 mm²/m

Required reinforcement ratio

ρy=Asb·dy=299.61000·110=0.00272

Provided reinforcement ratio

ρprov,y=As,provb·dy=314.21000·110=0.00286

Slab 20 - direction x (bottom)

Relative design moment

μEd,x2=MEd,x,2·10⁶b·dx2·fcd=12.63·10⁶1000·1202·16.67=0.0526

μEd,x2=0.0526 ≤ μl​im=0.295

✔ Single reinforcement is sufficient

Lever arm ratio z/d

ζx2=0.5·1+1−2·μEd,x2=0.5·1+1−2·0.0526=0.973

Required reinforcement

As,req,x2=MEd,x,2·10⁶ζx2·dx·fyd=12.63·10⁶0.973·120·434.8=248.8 mm²/m

Minimum reinforcement (EC2 9.2.1.1)

As,m​in,x2=max0.26·fctmfyk·b·dx0.0013·b·dx

=max0.26·2.56500·1000·1200.0013·1000·120=160.1 mm²/m

Provided reinforcement: 5Φ8/20 cm  →  A_s,prov = 251.3 mm²/m

Required reinforcement ratio

ρx2=Asb·dx=248.81000·120=0.00207

Provided reinforcement ratio

ρprov,x2=As,provb·dx=251.31000·120=0.00209

Slab 20 - direction y (bottom)

Relative design moment

μEd,y2=MEd,y,2·10⁶b·dy2·fcd=10.77·10⁶1000·1102·16.67=0.0534

μEd,y2=0.0534 ≤ μl​im=0.295

✔ Single reinforcement is sufficient

Lever arm ratio z/d

ζy2=0.5·1+1−2·μEd,y2=0.5·1+1−2·0.0534=0.9725

Required reinforcement

As,req,y2=MEd,y,2·10⁶ζy2·dy·fyd=10.77·10⁶0.9725·110·434.8=231.6 mm²/m

Minimum reinforcement (EC2 9.2.1.1)

As,m​in,y2=max0.26·fctmfyk·b·dy0.0013·b·dy

=max0.26·2.56500·1000·1100.0013·1000·110=146.7 mm²/m

Provided reinforcement: 5Φ8/20 cm  →  A_s,prov = 251.3 mm²/m

Required reinforcement ratio

ρy2=Asb·dy=231.61000·110=0.00211

Provided reinforcement ratio

ρprov,y2=As,provb·dy=251.31000·110=0.00228

Over the supports on axes 5 and 6 (top)

Relative design moment

μEd,56=MEd,56·10⁶b·dx2·fcd=31.03·10⁶1000·1202·16.67=0.1293

μEd,56=0.1293 ≤ μl​im=0.295

✔ Single reinforcement is sufficient

Lever arm ratio z/d

ζ56=0.5·1+1−2·μEd,56=0.5·1+1−2·0.1293=0.9305

Required reinforcement

As,req,56=MEd,56·10⁶ζ56·dx·fyd=31.03·10⁶0.9305·120·434.8=639.2 mm²/m

Minimum reinforcement (EC2 9.2.1.1)

As,m​in,56=max0.26·fctmfyk·b·dx0.0013·b·dx

=max0.26·2.56500·1000·1200.0013·1000·120=160.1 mm²/m

Provided reinforcement: Φ12/17.5 cm  →  A_s,prov = 646.3 mm²/m

Required reinforcement ratio

ρ56=Asb·dx=639.21000·120=0.00533

Provided reinforcement ratio

ρprov,56=As,provb·dx=646.31000·120=0.00539

Balconies - top reinforcement at the root of the cantilever:

Relative design moment

μEd,c=MEd,c·10⁶b·dx2·fcd=25.7·10⁶1000·1202·16.67=0.1071

μEd,c=0.1071 ≤ μl​im=0.295

✔ Single reinforcement is sufficient

Lever arm ratio z/d

ζc=0.5·1+1−2·μEd,c=0.5·1+1−2·0.1071=0.9432

Required reinforcement

As,req,c=MEd,c·10⁶ζc·dx·fyd=25.7·10⁶0.9432·120·434.8=522.2 mm²/m

Minimum reinforcement (EC2 9.2.1.1)

As,m​in,c=max0.26·fctmfyk·b·dx0.0013·b·dx

=max0.26·2.56500·1000·1200.0013·1000·120=160.1 mm²/m

Provided reinforcement: Φ10/15 cm  →  A_s,prov = 523.6 mm²/m

Required reinforcement ratio

ρc=Asb·dx=522.21000·120=0.00435

Provided reinforcement ratio

ρprov,c=As,provb·dx=523.61000·120=0.00436

## 5. Design for shear

V_Ed,max = max(V_Ed,x; V_Ed,y; V_Ed,c) = 30.4 kN/m

Size factor

k=min1+200dx2=min1+2001202=2

vm​in=0.035·k32·fck=0.035·232·25=0.495 N/mm²

Shear resistance without shear reinforcement (EC2 6.2.2)

VRd,c=CRd,c·k·3100·ρl·fck·b·dx10³=0.12·2·3100·0.00436·25·1000·12010³=63.87 kN/m

VRd,c,m​in=vm​in·b·dx10³=0.495·1000·12010³=59.4 kN/m

V_Rd,c = max(V_Rd,c ; V_Rd,c,min) = 63.87 kN/m

VEd,m​ax=30.4 kN/m ≤ VRd,c=63.87 kN/m

✔ No shear reinforcement is required

## 6. Control of deflection (EC2 7.4.2)

Two-way slab - the shorter span is checked; one edge continuous: K = 1.3

ρ0=fck·10⁻³=25·10⁻³=0.005

Basic span/depth ratio, ρ ≤ ρ_0 (EC2 7.16a)

(l/d)basic=K·11+1.5·fck·ρ0ρ+3.2·fck·ρ0ρ−132

=1.3·11+1.5·25·0.0050.00272+3.2·25·0.0050.00272−132=48.1

Steel stress under the quasi-permanent load

σs=fyd·gk+0.3·qkgd+qd·As,reqAs,prov=434.8·5.21+0.3·3.57.034+5.25·299.6314.2=211.3 N/mm²

Correction factor 310/σ_s (≤ 1.5)

kσ=min310σs1.5=min310211.31.5=1.467

(l/d)l​im=(l/d)basic·kσ=48.1·1.467=70.57

l/d=leffdy=5700110=51.82

l/d=51.82 ≤ (l/d)l​im=70.57

✔ Deflection need not be calculated (l/d check satisfied)

Balcony (cantilever): K = 0.4

ρ0=fck·10⁻³=25·10⁻³=0.005

Basic span/depth ratio, ρ ≤ ρ_0 (EC2 7.16a)

(l/d)basic=K·11+1.5·fck·ρ0ρ+3.2·fck·ρ0ρ−132

=0.4·11+1.5·25·0.0050.00435+3.2·25·0.0050.00435−132=8.215

Steel stress under the quasi-permanent load

σs=fyd·gk+0.3·qkgd+qd·As,reqAs,prov=434.8·5.655+0.3·37.634+4.5·522.2523.6=234.3 N/mm²

Correction factor 310/σ_s (≤ 1.5)

kσ=min310σs1.5=min310234.31.5=1.323

(l/d)l​im=(l/d)basic·kσ=8.215·1.323=10.87

l/d=leffdx=1825120=15.21

l/d=15.21 > (l/d)l​im=10.87

✘ l/d too large - increase the depth

## 7. Reinforcement summary

Table 10. Reinforcement of the slabs in two directions

| Location | M_Ed [kNm/m] | A_s,req [mm²/m] | Adopted | A_s,prov [mm²/m] |
| --- | --- | --- | --- | --- |
| 19/21 x | 13.97 | 276 | Φ8/17.5 cm | 287.2 |
| 19/21 y | 13.82 | 299.6 | Φ8/16 cm | 314.2 |
| 20 x | 12.63 | 248.8 | 5Φ8/20 cm | 251.3 |
| 20 y | 10.77 | 231.6 | 5Φ8/20 cm | 251.3 |
| supports 5, 6 | 31.03 | 639.2 | Φ12/17.5 cm | 646.3 |
| balcony root | 25.7 | 522.2 | Φ10/15 cm | 523.6 |

Note: the simplified l/d check of the cantilever is not satisfied. The balconies are made with a root thickness h_f and a tip thickness of 12 cm; increase the root thickness or verify the deflection by calculation (EC2 7.4.3).

# V. Design of reinforced concrete stairs

Two-flight staircase between axes 3 and 4. Storey height H = h_b = 290 cm, intermediate landing at H/2. Width of a flight d = 170 cm.

## 1. Geometry and static scheme

Height of the storey served by the stair

H=290 cm

Height of one flight

Hfl=H2=2902=145 cm

Number of risers in a flight (riser ≤ 17.5 cm)

n=9

Height of a riser

hst=Hfln=1459=16.11 cm

Width of a tread (2·h_st + b_st ≈ 62-64 cm)

bst=30 cm

2hst+b_st=2·hst+bst=2·16.11+30=62.22 cm

Horizontal projection of the flight

lfl=n−1·bst=9−1·30=240 cm

Depth of the landing (= d)

lla=170 cm

Inclination of the flight

α=arctanhstbst=arctan16.1130=28.24 °

Thickness of the waist slab

h=17 cm

Span of the simply supported model (flight + landing)

leff=lfl+lla=240+170=410 cm

Figure 8. Flight and landing - simply supported inclined slab

## 2. Actions (per 1 m width)

Flight (per m² of plan):

Waist slab (h/cos α) and steps (h_st/2)

gk,1=hcos α·25+hst2·25=0.170.881·25+0.16112·25=6.838 kN/m²

Plaster

gk,2=0.02cos α·18=0.020.881·18=0.4086 kN/m²

Finishing of the steps

gk,3=1 kN/m²

gk,fl=gk,1+gk,2+gk,3=6.838+0.4086+1=8.247 kN/m²

Landing:

gk,la=h·25+0.02·18+1=0.17·25+0.02·18+1=5.61 kN/m²

Imposed load on stairs

qk=3 kN/m²

Design load on the flight

pfl=1.35·gk,fl+1.5·qk=1.35·8.247+1.5·3=15.63 kN/m

Design load on the landing

pla=1.35·gk,la+1.5·qk=1.35·5.61+1.5·3=12.07 kN/m

## 3. Static analysis

Reaction at the lower support

RA=pfl·lfl·leff−lfl2+pla·lla22leff=15.63·2.4·4.1−2.42+12.07·1.7224.1=30.79 kN/m

RB=pfl·lfl+pla·lla−RA=15.63·2.4+12.07·1.7−30.79=27.25 kN/m

Section with V = 0 (on the flight)

x0=RApfl=30.7915.63=1.97 m

maxMEd=RA22·pfl=30.7922·15.63=30.33 kNm/m

maxVEd=maxRARB=max30.7927.25=30.79 kN/m

Figure 9. Bending moment and shear force in the stair

## 4. Design of the section

Design characteristics of the materials (EN 1992-1-1):

Concrete C25/30, reinforcing steel B500.

Characteristic cylinder strength of concrete

fck=25 N/mm²

Design compressive strength

fcd=αcc·fckγc=1·251.5=16.67 N/mm²

Mean tensile strength

fctm=2.56 N/mm²

Characteristic yield strength of steel

fyk=500 N/mm²

Design yield strength of steel

fyd=fykγs=5001.15=434.8 N/mm²

d=h−cnom−φ2=170−25−122=139 mm

Bottom reinforcement of the flight

Relative design moment

μEd=maxMEd·10⁶b·d2·fcd=30.33·10⁶1000·1392·16.67=0.0942

μEd=0.0942 ≤ μl​im=0.295

✔ Single reinforcement is sufficient

Lever arm ratio z/d

ζ=0.5·1+1−2·μEd=0.5·1+1−2·0.0942=0.9505

Required reinforcement

As,req=maxMEd·10⁶ζ·d·fyd=30.33·10⁶0.9505·139·434.8=528 mm²/m

Minimum reinforcement (EC2 9.2.1.1)

As,m​in=max0.26·fctmfyk·b·d0.0013·b·d

=max0.26·2.56500·1000·1390.0013·1000·139=185.4 mm²/m

Provided reinforcement: Φ10/14 cm  →  A_s,prov = 561 mm²/m

Required reinforcement ratio

ρ=Asb·d=5281000·139=0.0038

Provided reinforcement ratio

ρprov=As,provb·d=5611000·139=0.00404

## 5. Shear

Size factor

k=min1+200d2=min1+2001392=2

vm​in=0.035·k32·fck=0.035·232·25=0.495 N/mm²

Shear resistance without shear reinforcement (EC2 6.2.2)

VRd,c=CRd,c·k·3100·ρl·fck·b·d10³=0.12·2·3100·0.00404·25·1000·13910³=72.09 kN/m

VRd,c,m​in=vm​in·b·d10³=0.495·1000·13910³=68.8 kN/m

V_Rd,c = max(V_Rd,c ; V_Rd,c,min) = 72.09 kN/m

maxVEd=30.79 kN/m ≤ VRd,c=72.09 kN/m

✔ No shear reinforcement is required

## 6. Control of deflection

ρ0=fck·10⁻³=25·10⁻³=0.005

Basic span/depth ratio, ρ ≤ ρ_0 (EC2 7.16a)

(l/d)basic=K·11+1.5·fck·ρ0ρ+3.2·fck·ρ0ρ−132

=1·11+1.5·25·0.0050.0038+3.2·25·0.0050.0038−132=23.72

Steel stress under the quasi-permanent load

σs=fyd·gk+0.3·qkgd+qd·As,reqAs,prov=434.8·8.247+0.3·311.13+4.5·528561=239.4 N/mm²

Correction factor 310/σ_s (≤ 1.5)

kσ=min310σs1.5=min310239.41.5=1.295

(l/d)l​im=(l/d)basic·kσ=23.72·1.295=30.71

l/d=leffd=4100139=29.5

l/d=29.5 ≤ (l/d)l​im=30.71

✔ Deflection need not be calculated (l/d check satisfied)

## 7. Detailing

Distribution bars Φ8/20 cm in every step and along the waist; top bars Φ8/20 cm at both ends (0.2·l), bottom bars bent at the kink between flight and landing are replaced by two crossing bars anchored with l_bd.

# VI. Design of reinforced concrete simply supported beam

Beam B21 on axis 5 between axes A and B (offices). It carries the slabs 19 and 20 (two-way, h_f = 15 cm), its self-weight and a brick partition wall.

## 1. Static scheme and geometry

bw=25 cm

h=60 cm

hf=15 cm

Clear span (columns 25 cm on axes A and B)

lcl=555 cm

a_i = min(h/2; t/2)

leff=lcl+2·minh2t2=555+2·min602252=580 cm

The beam is on the short side of the slabs → triangular load (lines at 45°).

## 2. Actions

Characteristic permanent actions directly on the beam:

Self-weight of the beam

gk,1=bw·h·25=0.25·0.6·25=3.75 kN/m

Brick wall on the beam

gk,2=tw·hs−h·γw=0.25·3.1−0.6·17=10.62 kN/m

Plaster of the wall

gk,3=2·0.02·hs−hf·18=2·0.02·3.1−0.15·18=2.124 kN/m

ΣGk=gk,1+gk,2+gk,3=3.75+10.62+2.124=16.5 kN/m

From the slabs 19 and 20 (peak value of the triangle):

gfk=gk,slab·lload2=5.21·5.82=15.11 kN/m

qfk=qk,slab·lload2=3.5·5.82=10.15 kN/m

Design values:

gd=1.35·ΣGk=1.35·16.5=22.27 kN/m

gfd=1.35·gfk=1.35·15.11=20.4 kN/m

qfd=1.5·qfk=1.5·10.15=15.23 kN/m

## 3. Static analysis

UDL: g·l²/8 ; triangle (both sides): 2·w_0·l²/12

maxMEd=gd·leff28+2·gfd+qfd·leff212=22.27·5.828+2·20.4+15.23·5.8212=293.4 kNm

maxVEd=gd·leff2+2·gfd+qfd·leff4=22.27·5.82+2·20.4+15.23·5.84=167.9 kN

Quasi-permanent combination (ψ_2 = 0.3)

Mqp=ΣGk·leff28+2·gfk+0.3·qfk·leff212=16.5·5.828+2·15.11+0.3·10.15·5.8212=171.2 kNm

Bending moment at x = l/4 (from the analysis)

MΔx=M(l/4)=207.6 kNm

Reaction from permanent actions (design) - for the columns

Rg=gd·leff2+2·gfd·leff4=22.27·5.82+2·20.4·5.84=123.7 kN

Reaction from variable actions (design)

Rq=2·qfd·leff4=2·15.23·5.84=44.15 kN

Figure 10. Static scheme of beam B21 (plus the triangular/trapezoidal load from the slabs)

Figure 11. Bending moment and shear force diagrams of beam B21

## 4. Design of the bending reinforcement

Design characteristics of the materials (EN 1992-1-1):

Concrete C25/30, reinforcing steel B500.

Characteristic cylinder strength of concrete

fck=25 N/mm²

Design compressive strength

fcd=αcc·fckγc=1·251.5=16.67 N/mm²

Mean tensile strength

fctm=2.56 N/mm²

Characteristic yield strength of steel

fyk=500 N/mm²

Design yield strength of steel

fyd=fykγs=5001.15=434.8 N/mm²

Effective depth (c_nom = 25 mm, links Φ8, main bars Φ20)

d=h−cnom−φw−φ2=600−25−8−202=557 mm

Effective width of the flange (T-section, EC2 5.3.2.1):

b_eff,i = 0.2·b_i + 0.1·l_0 ≤ 0.2·l_0 ; ≤ b_i

beff,1=min0.2·b1+0.1·l00.2·l0b1=min0.2·3.025+0.1·5.80.2·5.83.025=1.16 m

b_eff,i = 0.2·b_i + 0.1·l_0 ≤ 0.2·l_0 ; ≤ b_i

beff,2=min0.2·b2+0.1·l00.2·l0b2=min0.2·3.025+0.1·5.80.2·5.83.025=1.16 m

Effective width of the flange

beff=bw+beff,1+beff,2=0.25+1.16+1.16=2.57 m

Compression force of the whole flange (0.8x = h_f)

Fcd=beff·hf·fcd10³=2570·150·16.6710³=6425 kN

Moment resisted by the flange

Mf=Fcd·d−hf210³=6425·557−150210³=3097 kNm

maxMEd=293.4 kNm ≤ Mf=3097 kNm

✔ The neutral axis is in the flange - rectangular section b = b_eff

Mid-span section

Relative design moment

μEd=maxMEd·10⁶beff·d2·fcd=293.4·10⁶2570·5572·16.67=0.0221

μEd=0.0221 ≤ μl​im=0.295

✔ Single reinforcement is sufficient

Lever arm ratio z/d

ζ=0.5·1+1−2·μEd=0.5·1+1−2·0.0221=0.9888

Required reinforcement

As,req=maxMEd·10⁶ζ·d·fyd=293.4·10⁶0.9888·557·434.8=1225 mm²

Minimum reinforcement (EC2 9.2.1.1)

As,m​in=max0.26·fctmfyk·bw·d0.0013·bw·d=max0.26·2.56500·250·5570.0013·250·557=185.7 mm²

Provided reinforcement: 4Φ20  →  A_s,prov = 1257 mm²

Required reinforcement ratio

ρ=Asbw·d=1225250·557=0.0088

Provided reinforcement ratio

ρprov=As,provbw·d=1257250·557=0.00902

## 5. Design for shear

Strut inclination θ: cot θ = 2.5  (θ = 21.8°)

Internal lever arm

z=0.9·d=0.9·557=501.3 mm

ν1=0.6·1−fck250=0.6·1−25250=0.54

Resistance of the compression struts (EC2 6.9)

VRd,m​ax=bw·z·ν1·fcdcot θ+1cot θ10³=250·501.3·0.54·16.672.5+12.510³=388.9 kN

maxVEd=167.9 kN ≤ VRd,m​ax=388.9 kN

✔ The concrete struts are not crushed

Size factor

k=min1+200d2=min1+2005572=1.599

vm​in=0.035·k32·fck=0.035·1.59932·25=0.3539 N/mm²

Shear resistance without shear reinforcement (EC2 6.2.2)

VRd,c=CRd,c·k·3100·ρl·fck·bw·d10³=0.12·1.599·3100·0.02·25·250·55710³=98.45 kN

VRd,c,m​in=vm​in·bw·d10³=0.3539·250·55710³=49.28 kN

V_Rd,c = max(V_Rd,c ; V_Rd,c,min) = 98.45 kN

maxVEd=167.9 kN > VRd,c=98.45 kN

➜ Shear reinforcement is required - designed below

Required shear reinforcement (EC2 6.8)

Asw/s=maxVEd·10³z·fyd·cot θ=167.9·10³501.3·434.8·2.5=0.3081 mm²/mm

ρw,m​in=0.08·fckfyk=0.08·25500=0.0008

(Asw/s)_m​in=ρw,m​in·bw=0.0008·250=0.2 mm²/mm

Spacing of 2-leg links Φ8 (A_sw = 2·50.27 mm²)

s=Asw(Asw/s)=100.50.3081=326.3 mm

Maximum longitudinal spacing of links (EC2 9.2.2)

sl,m​ax=0.75·d=0.75·557=417.8 mm

Adopted links: Φ8/30 cm (2-leg) in the support zones

Additional tensile force in the longitudinal reinforcement (EC2 6.2.3(7))

ΔFtd=0.5·maxVEd·cot θ=0.5·167.9·2.5=209.9 kN

Shear between the web and the flange (EC2 6.2.4):

Change of the longitudinal force in one flange outstand over Δx

ΔFd=MΔx·10⁶d−hf2·0.5·beff−bwbeff10³=207.6·10⁶557−1502·0.5·2570−250257010³=194.4 kN

Longitudinal shear stress

vEd=ΔFd·10³hf·Δx=194.4·10³150·1450=0.8936 N/mm²

vEd=0.8936 N/mm² > 0.4·fctd=0.4788

➜ Transverse reinforcement for the flange shear is needed - designed below

vEd=0.8936 N/mm² ≤ ν·fcd·sinθ·cosθ=3.6

✔ The compression struts in the flange are adequate

Required transverse reinforcement in the flange

Asf/s_f=vEd·hffyd·cot θf·1000=0.8936·150434.8·2·1000=154.1 mm²/m

## 6. Deflection of the beam (EC2 7.4.3)

Ecm=31.48 GPa

Effective modulus (creep)

Ec,eff=Ecm1+φ(∞,t0)=31.481+2=10.49 GPa

αe=EsEc,eff=20010.49=19.06

Uncracked section (state I):

AI=521954 mm²

xI=161.8 mm

II=14380563347 mm⁴

Cracking moment

Mcr=fctm·IIh−xI10⁶=2.56·14380563347600−161.810⁶=84.17 kNm

Cracked section (state II):

xII=93 mm

III=5846339912 mm⁴

Distribution coefficient (EC2 7.19)

ζ=1−β·McrMqp2=1−0.5·84.17171.22=0.8791

Curvature from the quasi-permanent moment

1/rm=Mqp·10⁶Ec,eff·1000·ζIII+1−ζII

=171.2·10⁶10.49·1000·0.87915846339912+1−0.879114380563347=0.00000259 1/mm

εcd=kh·εcd,0=0.903·0.00045=0.000406

εca=2.5·fck−10·10⁻⁶=2.5·25−10·10⁻⁶=0.000037

Total shrinkage strain

εcs=εcd+εca=0.000406+0.000037=0.000444

Curvature from shrinkage (EC2 7.21)

1/rcs=εcs·αe·ζ·SIIIII+1−ζ·SIII

=0.000444·19.06·0.8791·5830765846339912+1−0.8791·49664614380563347=0.000000777 1/mm

Deflection (k = 5/48)

f=5/48·1/rm+1/rcs·leff2=5/48·0.00000259+0.000000777·58002=11.8 mm

fl​im=leff250=5800250=23.2 mm

f=11.8 mm ≤ fl​im=23.2 mm

✔ The deflection is acceptable

# VII. Design of reinforced concrete beam for torsion

Beam on axis E (Д) between axes 3 and 4 carrying the cantilever canopy slab 7 (length 1.25·d = 212.5 cm). Bending: simply supported beam. Torsion: both ends fixed against rotation (walls on axes 3 and 4).

## 1. Static scheme

b=25 cm

h=50 cm

Clear span between the walls (2d + 45 − 25)

lcl=360 cm

Effective span for bending

leff=lcl+2·minh2t2=360+2·min502252=385 cm

Effective span for torsion: l_T = l_cl = 360 cm

Length of the cantilever measured to the axis of the beam

lc=1.25·d+b=2.125+0.125=2.25 m

## 2. Actions

Canopy slab 7 - characteristic permanent actions:

slab + plaster + screed 4 cm + insulation

Σgk7=hf7·25+0.02·18+0.04·20+0.2=0.15·25+0.02·18+0.04·20+0.2=5.11 kN/m²

Snow

sk=1 kN/m²

Torsional moment per metre (permanent)

Mcg=Σgk7·lc22=5.11·2.2522=12.93 kNm/m

Rcg=Σgk7·lc=5.11·2.25=11.5 kN/m

Msk=sk·lc22=1·2.2522=2.531 kNm/m

Rsk=sk·lc=1·2.25=2.25 kN/m

Self-weight and plaster of the beam

gk,b=b·h·25+0.02·18·2·h+b=0.25·0.5·25+0.02·18·2·0.5+0.25=3.575 kN/m

gd=1.35·Rcg+gk,b=1.35·11.5+3.575=20.35 kN/m

qd=1.5·Rsk=1.5·2.25=3.375 kN/m

pd=gd+qd=20.35+3.375=23.72 kN/m

Distributed torsional moment

td=1.35·Mcg+1.5·Msk=1.35·12.93+1.5·2.531=21.26 kNm/m

## 3. Static analysis

MEd=pd·leff28=23.72·3.8528=43.95 kNm

VEd=pd·leff2=23.72·3.852=45.67 kN

Torsion - beam fixed at both ends

TEd=td·lT2=21.26·3.62=38.27 kNm

Figure 12. Internal forces of the beam subjected to torsion

## 4. Design of the cross-section for torsion (EC2 6.3)

Design characteristics of the materials (EN 1992-1-1):

Concrete C25/30, reinforcing steel B500.

Characteristic cylinder strength of concrete

fck=25 N/mm²

Design compressive strength

fcd=αcc·fckγc=1·251.5=16.67 N/mm²

Mean tensile strength

fctm=2.56 N/mm²

Characteristic yield strength of steel

fyk=500 N/mm²

Design yield strength of steel

fyd=fykγs=5001.15=434.8 N/mm²

A=b·h=250·500=125000 mm²

u=2·b+h=2·250+500=1500 mm

Effective wall thickness

tef=Au=1250001500=83.33 mm

Ak=b−tef·h−tef=250−83.33·500−83.33=69444 mm²

uk=2·b−tef+h−tef=2·250−83.33+500−83.33=1167 mm

Effective depth (c_nom = 25 mm, links Φ8, main bars Φ16)

d=h−cnom−φw−φ2=500−25−8−162=459 mm

ν=0.6·1−fck250=0.6·1−25250=0.54

Angle of the compression struts θ = 45° (cot θ = 1).

T_Rd,max = 2·ν·α_cw·f_cd·A_k·t_ef·sinθ·cosθ

TRd,m​ax=2·ν·fcd·Ak·tef·0.510⁶=2·0.54·16.67·69444·83.33·0.510⁶=52.08 kNm

V_Rd,max = α_cw·b·z·ν·f_cd/(cotθ + tanθ)

VRd,m​ax=b·0.9·d·ν·fcd210³=250·0.9·459·0.54·16.67210³=464.7 kN

Interaction torsion + shear (EC2 6.29)

η=TEdTRd,m​ax+VEdVRd,m​ax=38.2752.08+45.67464.7=0.833

η=0.833 ≤ 1

✔ The concrete section is adequate

## 5. Reinforcement

Links for shear (both legs)

Asw/s,V=VEd·1000z·fyd·1=45.67·1000413.1·434.8·1=0.2543 mm²/mm

Links for torsion (one leg)

Asw/s,T=TEd·10⁶2·Ak·fyd·1=38.27·10⁶2·69444·434.8·1=0.6337 mm²/mm

Total, one leg

Asw1/s=Asw/s,V2+Asw/s,T=0.25432+0.6337=0.7608 mm²/mm

Maximum spacing of torsion links

sm​ax=minuk80.75·db=min116780.75·459250=145.8 mm

Adopted closed links: Φ10/10 cm

Longitudinal reinforcement for torsion (cot θ = 1)

ΣAsl=TEd·10⁶·uk2·Ak·fyd=38.27·10⁶·11672·69444·434.8=739.3 mm²

Bending reinforcement (rectangular section):

Relative design moment

μEd=MEd·10⁶b·d2·fcd=43.95·10⁶250·4592·16.67=0.0501

μEd=0.0501 ≤ μl​im=0.295

✔ Single reinforcement is sufficient

Lever arm ratio z/d

ζ=0.5·1+1−2·μEd=0.5·1+1−2·0.0501=0.9743

Required reinforcement

As,req=MEd·10⁶ζ·d·fyd=43.95·10⁶0.9743·459·434.8=226.1 mm²

Minimum reinforcement (EC2 9.2.1.1)

As,m​in=max0.26·fctmfyk·b·d0.0013·b·d=max0.26·2.56500·250·4590.0013·250·459=153.1 mm²

Provided reinforcement: 2Φ12  →  A_s,prov = 226.2 mm²

Required reinforcement ratio

ρ=Asb·d=226.1250·459=0.00197

Provided reinforcement ratio

ρprov=As,provb·d=226.2250·459=0.00197

Bottom face: bending + torsion share

As,bot=As+ΣAsl·bkuk=226.1+739.3·166.71167=331.7 mm²

Top face

As,top=ΣAsl·bkuk=739.3·166.71167=105.6 mm²

Each side face

As,side=ΣAsl·hkuk=739.3·416.71167=264 mm²

Adopted: bottom 3Φ12 (339.3 mm²), top 2Φ12 (226.2 mm²), side bars 3Φ12 on each face.

## 6. Canopy slab 7 - top reinforcement at the root of the cantilever

Design moment at the root (per metre)

MEd,7=1.35·Mcg+1.5·Msk=1.35·12.93+1.5·2.531=21.26 kNm/m

d7=h−cnom−φ2=150−25−102=120 mm

Relative design moment

μEd,7=MEd,7·10⁶b·d72·fcd=21.26·10⁶1000·1202·16.67=0.0886

μEd,7=0.0886 ≤ μl​im=0.295

✔ Single reinforcement is sufficient

Lever arm ratio z/d

ζ7=0.5·1+1−2·μEd,7=0.5·1+1−2·0.0886=0.9536

Required reinforcement

As,req,7=MEd,7·10⁶ζ7·d7·fyd=21.26·10⁶0.9536·120·434.8=427.3 mm²/m

Minimum reinforcement (EC2 9.2.1.1)

As,m​in,7=max0.26·fctmfyk·b·d70.0013·b·d7

=max0.26·2.56500·1000·1200.0013·1000·120=160.1 mm²/m

Provided reinforcement: Φ10/17.5 cm  →  A_s,prov = 448.8 mm²/m

Required reinforcement ratio

ρ7=Asb·d7=427.31000·120=0.00356

Provided reinforcement ratio

ρprov,7=As,provb·d7=448.81000·120=0.00374

# VIII. Design of reinforced concrete continuous beams

Secondary beam B5 (and B6-B9) of the exhibition hall: continuous beam over two spans (axes 1-2-3), supported on the walls on axes 1 and 3 and on the frame beam B15 (axis 2). Load width = spacing of the beams = 251.7 cm.

## 1. Static scheme

bw=25 cm

h=45 cm

Clear span

lcl=602.5 cm

leff=lcl+minh2t12+t22=602.5+min452252+302=630 cm

## 2. Actions

Web of the beam

gk,1=bw·h−hf·25=0.25·0.45−0.12·25=2.062 kN/m

Plaster

gk,2=0.02·18·2·h−hf+bw=0.02·18·2·0.45−0.12+0.25=0.3276 kN/m

From the slab (load width s)

gk,3=gk,slab·s=4.46·2.517=11.22 kN/m

Σgk=gk,1+gk,2+gk,3=2.062+0.3276+11.22=13.61 kN/m

Variable action from the slab

qk=qk1·s=5.6·2.517=14.09 kN/m

gd=1.35·Σgk=1.35·13.61=18.38 kN/m

qd=1.5·qk=1.5·14.09=21.14 kN/m

## 3. Static analysis

Continuous beam with two equal spans; permanent load on both spans, variable load in the most unfavourable arrangement (stiffness method, envelopes).

Span moment (q on one span)

maxMEd,1=αg·gd+αq·qd·leff2=0.0698·18.38+0.0952·21.14·6.32=130.8 kNm

Moment at the middle support (q on both spans)

minMB=−αg·gd+αq·qd·leff2=−0.125·18.38+0.125·21.14·6.32=-196.1 kNm

Shear at the middle support (5/8·p·l)

maxVEd=0.625·gd+qd·leff=0.625·18.38+21.14·6.3=155.6 kN

Maximum reaction at the end support

VEd,A=RA,m​ax=101.7 kN

Reaction on the frame beam B15 (10/8·p·l)

maxRB=1.25·gd+qd·leff=1.25·18.38+21.14·6.3=311.2 kN

Characteristic reaction - permanent

RB,gk=1.25·Σgk·leff=1.25·13.61·6.3=107.2 kN

Characteristic reaction - variable

RB,qk=1.25·qk·leff=1.25·14.09·6.3=111 kN

Figure 13. Static scheme of the continuous beam B5

Figure 14. Envelopes of M and V of beam B5

## 4. Design of the bending reinforcement

Design characteristics of the materials (EN 1992-1-1):

Concrete C25/30, reinforcing steel B500.

Characteristic cylinder strength of concrete

fck=25 N/mm²

Design compressive strength

fcd=αcc·fckγc=1·251.5=16.67 N/mm²

Mean tensile strength

fctm=2.56 N/mm²

Characteristic yield strength of steel

fyk=500 N/mm²

Design yield strength of steel

fyd=fykγs=5001.15=434.8 N/mm²

Effective depth (c_nom = 25 mm, links Φ8, main bars Φ20)

d=h−cnom−φw−φ2=450−25−8−202=407 mm

Spans - T-section, effective width (l_0 = 0.85·l_eff):

b_eff,i = 0.2·b_i + 0.1·l_0 ≤ 0.2·l_0 ; ≤ b_i

beff,1=min0.2·b1+0.1·l00.2·l0b1=min0.2·1.133+0.1·5.3550.2·5.3551.133=0.7622 m

b_eff,i = 0.2·b_i + 0.1·l_0 ≤ 0.2·l_0 ; ≤ b_i

beff,2=min0.2·b2+0.1·l00.2·l0b2=min0.2·1.133+0.1·5.3550.2·5.3551.133=0.7622 m

Effective width of the flange

beff=bw+beff,1+beff,2=0.25+0.7622+0.7622=1.774 m

Compression force of the whole flange (0.8x = h_f)

Fcd=beff·hf·fcd10³=1774·120·16.6710³=3549 kN

Moment resisted by the flange

Mf=Fcd·d−hf210³=3549·407−120210³=1231 kNm

maxMEd,1=130.8 kNm ≤ Mf=1231 kNm

✔ The neutral axis is in the flange - rectangular section b = b_eff

Span 1-2 and 2-3 (bottom)

Relative design moment

μEd,1=maxMEd,1·10⁶beff·d2·fcd=130.8·10⁶1774·4072·16.67=0.0267

μEd,1=0.0267 ≤ μl​im=0.295

✔ Single reinforcement is sufficient

Lever arm ratio z/d

ζ1=0.5·1+1−2·μEd,1=0.5·1+1−2·0.0267=0.9865

Required reinforcement

As,req,1=maxMEd,1·10⁶ζ1·d·fyd=130.8·10⁶0.9865·407·434.8=749.5 mm²

Minimum reinforcement (EC2 9.2.1.1)

As,m​in,1=max0.26·fctmfyk·bw·d0.0013·bw·d

=max0.26·2.56500·250·4070.0013·250·407=135.7 mm²

Provided reinforcement: 2Φ22  →  A_s,prov = 760.3 mm²

Required reinforcement ratio

ρ1=Asbw·d=749.5250·407=0.00737

Provided reinforcement ratio

ρprov,1=As,provbw·d=760.3250·407=0.00747

Middle support - rectangular section b_w (flange in tension), moment at the face:

ΔM = V_Ed·t_2/2

ΔM=VEd·t2200=155.6·30200=23.34 kNm

MEd,B=minMB−ΔM=-196.1−23.34=172.7 kNm

Relative design moment

μEd,B=MEd,B·10⁶bw·d2·fcd=172.7·10⁶250·4072·16.67=0.2503

μEd,B=0.2503 ≤ μl​im=0.295

✔ Single reinforcement is sufficient

Lever arm ratio z/d

ζB=0.5·1+1−2·μEd,B=0.5·1+1−2·0.2503=0.8534

Required reinforcement

As,req,B=MEd,B·10⁶ζB·d·fyd=172.7·10⁶0.8534·407·434.8=1144 mm²

Minimum reinforcement (EC2 9.2.1.1)

As,m​in,B=max0.26·fctmfyk·bw·d0.0013·bw·d

=max0.26·2.56500·250·4070.0013·250·407=135.7 mm²

Provided reinforcement: 4Φ20  →  A_s,prov = 1257 mm²

Required reinforcement ratio

ρB=Asbw·d=1144250·407=0.0112

Provided reinforcement ratio

ρprov,B=As,provbw·d=1257250·407=0.0124

End supports: A_s3 ≥ 0.25·A_s,span = 190.1 mm² → 2Φ14; assembly bars in the span 2Φ10.

## 5. Design for shear

Strut inclination θ: cot θ = 2.5  (θ = 21.8°)

Internal lever arm

z=0.9·d=0.9·407=366.3 mm

ν1=0.6·1−fck250=0.6·1−25250=0.54

Resistance of the compression struts (EC2 6.9)

VRd,m​ax=bw·z·ν1·fcdcot θ+1cot θ10³=250·366.3·0.54·16.672.5+12.510³=284.2 kN

maxVEd=155.6 kN ≤ VRd,m​ax=284.2 kN

✔ The concrete struts are not crushed

Size factor

k=min1+200d2=min1+2004072=1.701

vm​in=0.035·k32·fck=0.035·1.70132·25=0.3882 N/mm²

Shear resistance without shear reinforcement (EC2 6.2.2)

VRd,c=CRd,c·k·3100·ρl·fck·bw·d10³=0.12·1.701·3100·0.01235·25·250·40710³=65.16 kN

VRd,c,m​in=vm​in·bw·d10³=0.3882·250·40710³=39.5 kN

V_Rd,c = max(V_Rd,c ; V_Rd,c,min) = 65.16 kN

maxVEd=155.6 kN > VRd,c=65.16 kN

➜ Shear reinforcement is required - designed below

Required shear reinforcement (EC2 6.8)

Asw/s=maxVEd·10³z·fyd·cot θ=155.6·10³366.3·434.8·2.5=0.3908 mm²/mm

ρw,m​in=0.08·fckfyk=0.08·25500=0.0008

(Asw/s)_m​in=ρw,m​in·bw=0.0008·250=0.2 mm²/mm

Spacing of 2-leg links Φ8 (A_sw = 2·50.27 mm²)

s=Asw(Asw/s)=100.50.3908=257.2 mm

Maximum longitudinal spacing of links (EC2 9.2.2)

sl,m​ax=0.75·d=0.75·407=305.2 mm

Adopted links: Φ8/25 cm (2-leg) in the support zones

Additional tensile force in the longitudinal reinforcement (EC2 6.2.3(7))

ΔFtd=0.5·maxVEd·cot θ=0.5·155.6·2.5=194.5 kN

Supporting (hanger) reinforcement in the intersection with the frame beam B15:

The whole reaction is hung up by links

Asw,hang=maxRB·1000fyd=311.2·1000434.8=715.8 mm²

Required 8 additional two-leg links Φ8 in the intersection zone (each side of B5).

Shear between the web and the flange:

Change of the longitudinal force in one flange outstand over Δx

ΔFd=MΔx·10⁶d−hf2·0.5·beff−bwbeff10³=111.1·10⁶407−1202·0.5·1774−250177410³=137.6 kN

Longitudinal shear stress

vEd=ΔFd·10³hf·Δx=137.6·10³120·1575=0.728 N/mm²

vEd=0.728 N/mm² > 0.4·fctd=0.4788

➜ Transverse reinforcement for the flange shear is needed - designed below

vEd=0.728 N/mm² ≤ ν·fcd·sinθ·cosθ=3.6

✔ The compression struts in the flange are adequate

Required transverse reinforcement in the flange

Asf/s_f=vEd·hffyd·cot θf·1000=0.728·120434.8·2·1000=100.5 mm²/m

## 6. Control of deflection

End span of a continuous beam: K = 1.3 (flanged section b_eff/b_w > 3 → × 0.8)

ρ0=fck·10⁻³=25·10⁻³=0.005

Basic span/depth ratio, ρ > ρ_0 (EC2 7.16b, ρ' = 0)

(l/d)basic=K·11+1.5·fck·ρ0ρ=1.04·11+1.5·25·0.0050.00737=16.73

Steel stress under the quasi-permanent load

σs=fyd·gk+0.3·qkgd+qd·As,reqAs,prov

=434.8·13.61+0.3·14.0918.38+21.14·749.5760.3=193.5 N/mm²

Correction factor 310/σ_s (≤ 1.5)

kσ=min310σs1.5=min310193.51.5=1.5

(l/d)l​im=(l/d)basic·kσ=16.73·1.5=25.1

l/d=leffd=6300407=15.48

l/d=15.48 ≤ (l/d)l​im=25.1

✔ Deflection need not be calculated (l/d check satisfied)

# IX. Design of reinforced concrete columns

Column C10 at the intersection of axes 5 and B (offices). It carries beam B21 (axis 5) and the continuous beam B12 (axis B, axes 4-7) on every floor, and the roof.

## 1. Static scheme

Cross-section 25/60 cm. Storeys: first h_b = 290 cm, then 4 × h_s = 310 cm. The column is part of a braced structure (walls) - effective length l_0 = 0.7·l.

## 2. Actions

### 2.1 From beam B21 (section VI)

Design reaction - permanent

Rgd,B21=123.7 kN

Design reaction - variable

Rqd,B21=44.15 kN

### 2.2 From the continuous beam B12 (axis B)

Self-weight of B12 + brick wall + plaster

gk,1=b·h·25+tw·hs−h·γw+2·0.02·hs−hf·18

=0.25·0.6·25+0.25·3.1−0.6·17+2·0.02·3.1−0.15·18=16.5 kN/m

From the corridor slab 17 (one-way, span c)

gk,2=gk,corr·c=4.96·1.75=8.68 kN/m

qk,2=qk·c=3.5·1.75=6.125 kN/m

Peak value from the office slabs (trapezoid)

gk,3=gk,off·ls2=5.21·5.82=15.11 kN/m

qk,3=qk·ls2=3.5·5.82=10.15 kN/m

Design reaction on axis 5 - permanent (stiffness method)

Rgd,B12=313.5 kN

Design reaction on axis 5 - variable, pattern loading for max R

Rqd,B12=134.1 kN

### 2.3 From the roof

Characteristic permanent actions on the roof (offices / corridor):

Table 11. Permanent actions - roof (offices / corridor)

| Layer | g_k [kN/m²] |
| --- | --- |
| waterproofing | 0.04 |
| protective layer | 0.66 |
| thermal insulation | 0.2 |
| vapour barrier | 0.03 |
| levelling cement screed | 0.44 |
| lightweight concrete for slope | 1.76 |
| RC slab h_f = 15 cm | 3.75 |
| ceiling plaster | 0.36 |

Total permanent action of the roof

Σgk,roof=7.24 kN/m²

Variable action on the roof (snow / maintenance)

sk=1 kN/m²

Roof beams B12 + B21 - permanent

Rgd,r=361.9 kN

Roof - snow

Rqd,r=50.93 kN

### 2.4 Self-weight of the column (one storey)

Gd,c=1.35·b·h·hs·25+2·0.02·b+h·hs·18

=1.35·0.25·0.6·3.1·25+2·0.02·0.25+0.6·3.1·18=18.25 kN

## 3. Static analysis - normal forces

Permanent - one floor

Ng1=Rgd,B21+Rgd,B12+Gd,c=123.7+313.5+18.25=455.5 kN

Variable - one floor

Nq1=Rqd,B21+Rqd,B12=44.15+134.1=178.3 kN

Permanent - roof

Ngr=Rgd,r+Gd,c=361.9+18.25=380.1 kN

Variable - roof

Nsr=Rqd,r=50.93 kN

The variable action of n floors is reduced by α_n = (2 + (n − 2)·ψ_0)/n, ψ_0 = 0.7 (EN 1991-1-1, 6.3.1.2(11)).

Table 12. Normal forces in column C10

| Section | Floors above | α_n | N_g [kN] | N_q [kN] | N_Ed [kN] |
| --- | --- | --- | --- | --- | --- |
| 4-4 | 0 | 1 | 380.1 | 50.93 | 431.1 |
| 3-3 | 1 | 1 | 835.6 | 229.2 | 1065 |
| 2-2 | 2 | 1 | 1291 | 407.4 | 1699 |
| 1-1 | 3 | 0.9 | 1747 | 532.2 | 2279 |
| 0-0 | 4 | 0.85 | 2202 | 657 | 2859 |

## 4. Design of the column

Design characteristics of the materials (EN 1992-1-1):

Concrete C25/30, reinforcing steel B500.

Characteristic cylinder strength of concrete

fck=25 N/mm²

Design compressive strength

fcd=αcc·fckγc=1·251.5=16.67 N/mm²

Mean tensile strength

fctm=2.56 N/mm²

Characteristic yield strength of steel

fyk=500 N/mm²

Design yield strength of steel

fyd=fykγs=5001.15=434.8 N/mm²

### 4.1 Section 0-0  (N_Ed = 2859 kN)

Preliminary design for the axial force (ρ = 1 %):

Ac,req=NEd·1000fcd+0.01·fyd=2859·100014.17+0.01·434.8=154431 mm²

EC2 9.5.2(2)

As,m​in=max0.1·NEd·1000fyd0.002·Ac=max0.1·2859·1000434.80.002·150000=657.6 mm²

Weak axis - bending about the weak axis: b = 600 mm, h = 250 mm

Effective length

l0=2.38 m

Radius of gyration

i=h12=25012=72.17 mm

Slenderness

λ=l0·1000i=2.38·100072.17=32.98

Relative normal force (f_cd with α_cc = 0.85)

n=NEd·1000Ac·fcd=2859·1000150000·14.17=1.346

EC2 5.8.3.1: A = 0.7, B = √(1+2ω), C = 1.7 − r_m

λl​im=20·A·B·Cn=20·0.7·1.569·0.71.346=13.25

Eccentricity from imperfections (EC2 6.1(4), 5.2(7))

ei=maxl0·1000400h3020=max2.38·10004002503020=20 mm

➜ λ > λ_lim - second order effects are taken into account (nominal curvature method, EC2 5.8.8)

K_r = (n_u − n)/(n_u − n_bal) ≤ 1

Kr=0.2895

K_φ = 1 + β·φ_ef, φ_ef = 2

Kφ=1.51

1/r0=fydEs·0.45·d=434.8200000·0.45·205=0.0000236 1/mm

1/r=Kr·Kφ·1/r0=0.2895·1.51·0.0000236=0.0000103 1/mm

Second order eccentricity

e2=1/r·l0·1000210=0.0000103·2.38·1000210=5.836 mm

M_Ed = max(M_0e; M_02) + N_Ed·e_i + N_Ed·e_2

MEd=maxM0e+NEd·ei1000M02+NEd·ei1000+NEd·e21000

=max0+2859·2010000+2859·201000+2859·5.8361000=73.87 kNm

Relative moment

μEd=MEd·10⁶b·h2·fcd=73.87·10⁶600·2502·14.17=0.139

Required reinforcement from the N-M interaction diagram of the section (symmetric reinforcement, d_1 = 45 mm, rectangular stress block, ε_cu = 3.5 ‰):

(ω used for λ_lim and K_r: ω = 0.7307 - iterated until it agrees with the result)

Mechanical reinforcement ratio (from the diagram)

ωtot=0.731

As,tot=ωtot·b·h·fcdfyd=0.731·600·250·14.17434.8=3572 mm²

A_s,tot = max(A_s,tot; A_s,min) = 3572 mm² → adopted 6Φ28 (3695 mm²), ρ = 2.463 % (0.2 % ≤ ρ ≤ 4 %)

### 4.2 Section 1-1  (N_Ed = 2279 kN)

Preliminary design for the axial force (ρ = 1 %):

Ac,req=NEd·1000fcd+0.01·fyd=2279·100014.17+0.01·434.8=123088 mm²

EC2 9.5.2(2)

As,m​in=max0.1·NEd·1000fyd0.002·Ac=max0.1·2279·1000434.80.002·150000=524.2 mm²

Weak axis - bending about the weak axis: b = 600 mm, h = 250 mm

Effective length

l0=2.17 m

Radius of gyration

i=h12=25012=72.17 mm

Slenderness

λ=l0·1000i=2.17·100072.17=30.07

Relative normal force (f_cd with α_cc = 0.85)

n=NEd·1000Ac·fcd=2279·1000150000·14.17=1.072

EC2 5.8.3.1: A = 0.7, B = √(1+2ω), C = 1.7 − r_m

λl​im=20·A·B·Cn=20·0.7·1.311·0.71.072=12.4

Eccentricity from imperfections (EC2 6.1(4), 5.2(7))

ei=maxl0·1000400h3020=max2.17·10004002503020=20 mm

➜ λ > λ_lim - second order effects are taken into account (nominal curvature method, EC2 5.8.8)

K_r = (n_u − n)/(n_u − n_bal) ≤ 1

Kr=0.2989

K_φ = 1 + β·φ_ef, φ_ef = 2

Kφ=1.549

1/r0=fydEs·0.45·d=434.8200000·0.45·205=0.0000236 1/mm

1/r=Kr·Kφ·1/r0=0.2989·1.549·0.0000236=0.0000109 1/mm

Second order eccentricity

e2=1/r·l0·1000210=0.0000109·2.17·1000210=5.139 mm

M_Ed = max(M_0e; M_02) + N_Ed·e_i + N_Ed·e_2

MEd=maxM0e+NEd·ei1000M02+NEd·ei1000+NEd·e21000

=max0+2279·2010000+2279·201000+2279·5.1391000=57.29 kNm

Relative moment

μEd=MEd·10⁶b·h2·fcd=57.29·10⁶600·2502·14.17=0.1078

Required reinforcement from the N-M interaction diagram of the section (symmetric reinforcement, d_1 = 45 mm, rectangular stress block, ε_cu = 3.5 ‰):

(ω used for λ_lim and K_r: ω = 0.3591 - iterated until it agrees with the result)

Mechanical reinforcement ratio (from the diagram)

ωtot=0.359

As,tot=ωtot·b·h·fcdfyd=0.359·600·250·14.17434.8=1756 mm²

A_s,tot = max(A_s,tot; A_s,min) = 1756 mm² → adopted 6Φ20 (1885 mm²), ρ = 1.257 % (0.2 % ≤ ρ ≤ 4 %)

Figure 15. Cross-section of column C10 (section 0-0)

Links: Φ8, spacing s ≤ min(20·Φ_long; b; 400 mm) = 250 mm, reduced by 0.6 within h above/below beams and at laps (EC2 9.5.3).

# X. Design of reinforced concrete frame

Frame on axis 2: beams B15 between axes A - C - E (two spans of 755 cm), columns C1 (A), C7 (C) and C12 (E). 5 storeys: h_b = 290 cm and 4 × h_s = 310 cm; fixed at the top of the footings.

## 1. Static scheme and stiffness

Beam B15: b_w/h = 30/65 cm, flange h_f = 12 cm, b_eff = 278.4 cm. Columns C1, C12: 40/60 cm; C7: 40/80 cm (h in the plane of the frame).

Moment of inertia of the T-section of the beam

Ib=1550090 cm⁴

Columns C1, C12

Ic,1=720000 cm⁴

Column C7

Ic,7=1706667 cm⁴

## 2. Actions (characteristic values)

### 2.1 Point loads from the secondary beams B5-B9 (section VIII)

Reaction R_B of B5 - permanent

Gk,1=107.2 kN

Reaction R_B of B5 - variable

Qk,1=111 kN

### 2.2 Facade beams on axes A and E (B1, B10) - reaction on axis 2

Beam 30/60 + facade wall + plaster + slab (s/2)

gk,f=b·h·25+b·hs−h·18+2·0.02·hs−hf·18+gk,slab·s

=0.3·0.6·25+0.3·3.1−0.6·18+2·0.02·3.1−0.12·18+4.46·1.258=25.76 kN/m

qk,f=qk1·s=5.6·1.258=7.047 kN/m

Middle reaction of a two-span beam (10/8·w·l)

Gk,2=1.25·gk,f·a=1.25·25.76·6.3=202.8 kN

Qk,2=1.25·qk,f·a=1.25·7.047·6.3=55.49 kN

### 2.3 Roof

Characteristic permanent actions on the roof above the hall:

Table 13. Permanent actions - roof above the hall

| Layer | g_k [kN/m²] |
| --- | --- |
| waterproofing | 0.04 |
| protective layer | 0.66 |
| thermal insulation | 0.2 |
| vapour barrier | 0.03 |
| levelling cement screed | 0.44 |
| lightweight concrete for slope | 1.76 |
| RC slab h_f = 12 cm | 3 |
| ceiling plaster | 0.36 |

Total permanent action of the roof

Σgk,roof=6.49 kN/m²

Variable action on the roof (snow / maintenance)

sk=1 kN/m²

Roof beams B5-B9

gk,r5=bw·h−hf·25+Σgk,roof·s=0.25·0.45−0.12·25+6.49·2.517=18.4 kN/m

Gkr,1=1.25·gk,r5·a=1.25·18.4·6.3=144.9 kN

Qkr,1=1.25·sk·s·a=1.25·1·2.517·6.3=19.82 kN

Roof facade beam + parapet 1.0 kN/m

Gkr,2=1.25·b·h·25+Σgk,roof·s+1·a=1.25·0.3·0.6·25+6.49·1.258+1·6.3=107.6 kN

Qkr,2=1.25·sk·s·a=1.25·1·1.258·6.3=9.909 kN

### 2.4 Frame beam B15 and columns

Web of B15 + plaster

gk,B15=bw·h−hf·25+0.02·18·2·h−hf+bw=0.3·0.65−0.12·25+0.02·18·2·0.65−0.12+0.3=4.465 kN/m

Self-weight of the columns: C1/C12: 6 kN/m, C7: 8 kN/m (applied at the top of every storey).

Load cases (design values 1.35·G + 1.5·Q):

LC1 - variable load on all spans of all floors (maximum normal forces and support moments)

LC2 - chequerboard: variable load in bay A-C on storeys 1, 3, 5 and in bay C-E on storeys 2, 4

LC3 - the opposite chequerboard (maximum span moments / column moments)

The frame is analysed by the stiffness method (Euler-Bernoulli elements, E_cm, gross sections, T-section for the beams).

Figure 16. Static scheme of the frame and characteristic loads from the secondary beams

Figure 17. Bending moment diagram - LC1

Figure 18. Bending moment diagram - LC2

Figure 19. Bending moment diagram - LC3

Figure 20. Shear force diagram - LC1

Figure 21. Normal force diagram - LC1

Table 14. Frame beam B15 - design values [kNm, kN]

| LC | Floor | max M_span | M at C7 | M at C1 | max ¦V¦ |
| --- | --- | --- | --- | --- | --- |
| LC1 | 1 | 340.1 | -612.8 | -415.5 | 360.1 |
| LC1 | 2 | 327.6 | -593.7 | -443.9 | 353.8 |
| LC1 | 3 | 327.4 | -588.3 | -446.9 | 352.7 |
| LC1 | 4 | 323.3 | -580.3 | -457 | 350.3 |
| LC1 | 5 | 275.9 | -458 | -264.9 | 273.6 |
| LC2 | 1 | 355 | -572.5 | -413.4 | 355 |
| LC2 | 2 | 354.1 | -553.6 | -230.8 | 351.1 |
| LC2 | 3 | 355.5 | -550.3 | -423.7 | 350.7 |
| LC2 | 4 | 345.3 | -543.6 | -243.2 | 347.4 |
| LC2 | 5 | 296.9 | -451.4 | -236.8 | 276.5 |
| LC3 | 1 | 355 | -572.5 | -202.3 | 355 |
| LC3 | 2 | 354.1 | -553.6 | -424.1 | 351.1 |
| LC3 | 3 | 355.5 | -550.3 | -232.3 | 350.7 |
| LC3 | 4 | 345.3 | -543.6 | -442.3 | 347.4 |
| LC3 | 5 | 296.9 | -451.4 | -229.4 | 276.5 |

Table 15. Columns - design forces

| LC | Column | Storey | N_Ed [kN] | M_bottom [kNm] | M_top [kNm] |
| --- | --- | --- | --- | --- | --- |
| LC1 | C1 | 1 | 3190 | 92.78 | -181.4 |
| LC1 | C1 | 2 | 2501 | 234.2 | -223.7 |
| LC1 | C7 | 1 | 5016 | -0.0000000000102 | 0.00000000000364 |
| LC1 | C7 | 2 | 3954 | -0.00000000000837 | 0.00000000000632 |
| LC2 | C1 | 1 | 2705 | 107.9 | -212 |
| LC2 | C1 | 2 | 2011 | 201.4 | -112.8 |
| LC2 | C7 | 1 | 3943 | -74.09 | 159.1 |
| LC2 | C7 | 2 | 3142 | -65.74 | -114.2 |
| LC3 | C1 | 1 | 2661 | 29.65 | -57.07 |
| LC3 | C1 | 2 | 2215 | 145.2 | -217.4 |
| LC3 | C7 | 1 | 3943 | 74.09 | -159.1 |
| LC3 | C7 | 2 | 3142 | 65.74 | 114.2 |

## 3. Design of the frame beam B15

Design characteristics of the materials (EN 1992-1-1):

Concrete C25/30, reinforcing steel B500.

Characteristic cylinder strength of concrete

fck=25 N/mm²

Design compressive strength

fcd=αcc·fckγc=1·251.5=16.67 N/mm²

Mean tensile strength

fctm=2.56 N/mm²

Characteristic yield strength of steel

fyk=500 N/mm²

Design yield strength of steel

fyd=fykγs=5001.15=434.8 N/mm²

Effective depth (c_nom = 25 mm, links Φ8, main bars Φ20)

d=h−cnom−φw−φ2=650−25−8−202=607 mm

Maximum span moment: LC2, floor 3.

Compression force of the whole flange (0.8x = h_f)

Fcd=beff·hf·fcd10³=2784·120·16.6710³=5567 kN

Moment resisted by the flange

Mf=Fcd·d−hf210³=5567·607−120210³=3045 kNm

maxMEd,span=355.5 kNm ≤ Mf=3045 kNm

✔ The neutral axis is in the flange - rectangular section b = b_eff

Span section (bottom)

Relative design moment

μEd,span=maxMEd,span·10⁶beff·d2·fcd=355.5·10⁶2784·6072·16.67=0.0208

μEd,span=0.0208 ≤ μl​im=0.295

✔ Single reinforcement is sufficient

Lever arm ratio z/d

ζspan=0.5·1+1−2·μEd,span=0.5·1+1−2·0.0208=0.9895

Required reinforcement

As,req,span=maxMEd,span·10⁶ζspan·d·fyd=355.5·10⁶0.9895·607·434.8=1361 mm²

Minimum reinforcement (EC2 9.2.1.1)

As,m​in,span=max0.26·fctmfyk·bw·d0.0013·bw·d

=max0.26·2.56500·300·6070.0013·300·607=242.9 mm²

Provided reinforcement: 3Φ25  →  A_s,prov = 1473 mm²

Required reinforcement ratio

ρspan=Asbw·d=1361300·607=0.00748

Provided reinforcement ratio

ρprov,span=As,provbw·d=1473300·607=0.00809

Support at column C7 (top) - LC1, floor 1; moment at the face:

ΔM = V_Ed·h_c/2

ΔM=VEd·hc7=360.1·0.4=144 kNm

MEd,C7=MC7−ΔM=612.8−144=468.7 kNm

Relative design moment

μEd,C7=MEd,C7·10⁶bw·d2·fcd=468.7·10⁶300·6072·16.67=0.2544

μEd,C7=0.2544 ≤ μl​im=0.295

✔ Single reinforcement is sufficient

Lever arm ratio z/d

ζC7=0.5·1+1−2·μEd,C7=0.5·1+1−2·0.2544=0.8504

Required reinforcement

As,req,C7=MEd,C7·10⁶ζC7·d·fyd=468.7·10⁶0.8504·607·434.8=2088 mm²

Minimum reinforcement (EC2 9.2.1.1)

As,m​in,C7=max0.26·fctmfyk·bw·d0.0013·bw·d

=max0.26·2.56500·300·6070.0013·300·607=242.9 mm²

Provided reinforcement: 5Φ25  →  A_s,prov = 2454 mm²

Required reinforcement ratio

ρC7=Asbw·d=2088300·607=0.0115

Provided reinforcement ratio

ρprov,C7=As,provbw·d=2454300·607=0.0135

End support at column C1 (top):

Relative design moment

μEd,C1=MC1·10⁶bw·d2·fcd=457·10⁶300·6072·16.67=0.2481

μEd,C1=0.2481 ≤ μl​im=0.295

✔ Single reinforcement is sufficient

Lever arm ratio z/d

ζC1=0.5·1+1−2·μEd,C1=0.5·1+1−2·0.2481=0.8549

Required reinforcement

As,req,C1=MC1·10⁶ζC1·d·fyd=457·10⁶0.8549·607·434.8=2026 mm²

Minimum reinforcement (EC2 9.2.1.1)

As,m​in,C1=max0.26·fctmfyk·bw·d0.0013·bw·d

=max0.26·2.56500·300·6070.0013·300·607=242.9 mm²

Provided reinforcement: 5Φ25  →  A_s,prov = 2454 mm²

Required reinforcement ratio

ρC1=Asbw·d=2026300·607=0.0111

Provided reinforcement ratio

ρprov,C1=As,provbw·d=2454300·607=0.0135

### Shear

Strut inclination θ: cot θ = 2.5  (θ = 21.8°)

Internal lever arm

z=0.9·d=0.9·607=546.3 mm

ν1=0.6·1−fck250=0.6·1−25250=0.54

Resistance of the compression struts (EC2 6.9)

VRd,m​ax=bw·z·ν1·fcdcot θ+1cot θ10³=300·546.3·0.54·16.672.5+12.510³=508.6 kN

VEd=360.1 kN ≤ VRd,m​ax=508.6 kN

✔ The concrete struts are not crushed

Size factor

k=min1+200d2=min1+2006072=1.574

vm​in=0.035·k32·fck=0.035·1.57432·25=0.3456 N/mm²

Shear resistance without shear reinforcement (EC2 6.2.2)

VRd,c=CRd,c·k·3100·ρl·fck·bw·d10³=0.12·1.574·3100·0.01348·25·300·60710³=111.1 kN

VRd,c,m​in=vm​in·bw·d10³=0.3456·300·60710³=62.93 kN

V_Rd,c = max(V_Rd,c ; V_Rd,c,min) = 111.1 kN

VEd=360.1 kN > VRd,c=111.1 kN

➜ Shear reinforcement is required - designed below

Required shear reinforcement (EC2 6.8)

Asw/s=VEd·10³z·fyd·cot θ=360.1·10³546.3·434.8·2.5=0.6064 mm²/mm

ρw,m​in=0.08·fckfyk=0.08·25500=0.0008

(Asw/s)_m​in=ρw,m​in·bw=0.0008·300=0.24 mm²/mm

Spacing of 2-leg links Φ10 (A_sw = 2·78.54 mm²)

s=Asw(Asw/s)=157.10.6064=259 mm

Maximum longitudinal spacing of links (EC2 9.2.2)

sl,m​ax=0.75·d=0.75·607=455.2 mm

Adopted links: Φ10/25 cm (2-leg) in the support zones

Additional tensile force in the longitudinal reinforcement (EC2 6.2.3(7))

ΔFtd=0.5·VEd·cot θ=0.5·360.1·2.5=450.1 kN

Hanger reinforcement for the reactions of B5-B9 (R = 1.35·G_k,1 + 1.5·Q_k,1):

RB5=1.35·Gk,1+1.5·Qk,1=1.35·107.2+1.5·111=311.2 kN

As,hang=RB5·1000fyd=311.2·1000434.8=715.8 mm²

→ 5 additional two-leg links Φ10 at each secondary beam (or bent-up bars 2Φ16 at 45°).

## 4. Design of the columns

### Column C7 - first storey (40/80 cm)

Flexibility of the restraint at the top (course formula)

k1=Icl8·Iblb=17066672908·1550090755=0.3583

Footing (minimum recommended value)

k2=0.1

Effective length in the plane of the frame (braced, EC2 5.15)

l0=0.5·l·1+k10.45+k1·1+k20.45+k2=0.5·2.9·1+0.35830.45+0.3583·1+0.10.45+0.1=1.894 m

Out of the plane of the frame

l0,out=0.7·l=0.7·2.9=2.03 m

Table 16. Design forces of column C7 (first storey)

| LC | N_Ed [kN] | M_01 [kNm] | M_02 [kNm] |
| --- | --- | --- | --- |
| LC1 | 5016 | 0.00000000000364 | -0.0000000000102 |
| LC2 | 3943 | -74.09 | 159.1 |
| LC3 | 3943 | 74.09 | -159.1 |

LC1:

C7 - bending in the plane of the frame: b = 400 mm, h = 800 mm

Effective length

l0=1.894 m

Radius of gyration

i=h12=80012=230.9 mm

Slenderness

λ=l0·1000i=1.894·1000230.9=8.2

Relative normal force (f_cd with α_cc = 0.85)

n=NEd·1000Ac·fcd=5016·1000320000·14.17=1.107

EC2 5.8.3.1: A = 0.7, B = √(1+2ω), C = 1.7 − r_m

λl​im=20·A·B·Cn=20·0.7·1.173·0.71.107=10.93

Eccentricity from imperfections (EC2 6.1(4), 5.2(7))

ei=maxl0·1000400h3020=max1.894·10004008003020=26.67 mm

✔ λ ≤ λ_lim - second order effects may be ignored

M_Ed = M_02 + N_Ed·e_i

MEd=M02+NEd·ei1000=0.0000000000102+5016·26.671000=133.8 kNm

Relative moment

μEd=MEd·10⁶b·h2·fcd=133.8·10⁶400·8002·14.17=0.0369

Required reinforcement from the N-M interaction diagram of the section (symmetric reinforcement, d_1 = 45 mm, rectangular stress block, ε_cu = 3.5 ‰):

(ω used for λ_lim and K_r: ω = 0.1877 - iterated until it agrees with the result)

Mechanical reinforcement ratio (from the diagram)

ωtot=0.188

As,tot=ωtot·b·h·fcdfyd=0.188·400·800·14.17434.8=1957 mm²

LC2:

C7 - bending in the plane of the frame: b = 400 mm, h = 800 mm

Effective length

l0=1.894 m

Radius of gyration

i=h12=80012=230.9 mm

Slenderness

λ=l0·1000i=1.894·1000230.9=8.2

Relative normal force (f_cd with α_cc = 0.85)

n=NEd·1000Ac·fcd=3943·1000320000·14.17=0.8699

EC2 5.8.3.1: A = 0.7, B = √(1+2ω), C = 1.7 − r_m

λl​im=20·A·B·Cn=20·0.7·1.06·2.1660.8699=34.44

Eccentricity from imperfections (EC2 6.1(4), 5.2(7))

ei=maxl0·1000400h3020=max1.894·10004008003020=26.67 mm

✔ λ ≤ λ_lim - second order effects may be ignored

M_Ed = M_02 + N_Ed·e_i

MEd=M02+NEd·ei1000=159.1+3943·26.671000=264.3 kNm

Relative moment

μEd=MEd·10⁶b·h2·fcd=264.3·10⁶400·8002·14.17=0.0729

Required reinforcement from the N-M interaction diagram of the section (symmetric reinforcement, d_1 = 45 mm, rectangular stress block, ε_cu = 3.5 ‰):

(ω used for λ_lim and K_r: ω = 0.0614 - iterated until it agrees with the result)

Mechanical reinforcement ratio (from the diagram)

ωtot=0.042

As,tot=ωtot·b·h·fcdfyd=0.042·400·800·14.17434.8=433.6 mm²

C7 - bending out of the plane of the frame (N_max): b = 800 mm, h = 400 mm

Effective length

l0=2.03 m

Radius of gyration

i=h12=40012=115.5 mm

Slenderness

λ=l0·1000i=2.03·1000115.5=17.58

Relative normal force (f_cd with α_cc = 0.85)

n=NEd·1000Ac·fcd=5016·1000320000·14.17=1.107

EC2 5.8.3.1: A = 0.7, B = √(1+2ω), C = 1.7 − r_m

λl​im=20·A·B·Cn=20·0.7·1.223·0.71.107=11.39

Eccentricity from imperfections (EC2 6.1(4), 5.2(7))

ei=maxl0·1000400h3020=max2.03·10004004003020=20 mm

➜ λ > λ_lim - second order effects are taken into account (nominal curvature method, EC2 5.8.8)

K_r = (n_u − n)/(n_u − n_bal) ≤ 1

Kr=0.1662

K_φ = 1 + β·φ_ef, φ_ef = 2

Kφ=1.716

1/r0=fydEs·0.45·d=434.8200000·0.45·355=0.0000136 1/mm

1/r=Kr·Kφ·1/r0=0.1662·1.716·0.0000136=0.0000039 1/mm

Second order eccentricity

e2=1/r·l0·1000210=0.0000039·2.03·1000210=1.599 mm

M_Ed = max(M_0e; M_02) + N_Ed·e_i + N_Ed·e_2

MEd=maxM0e+NEd·ei1000M02+NEd·ei1000+NEd·e21000

=max0+5016·2010000+5016·201000+5016·1.5991000=108.3 kNm

Relative moment

μEd=MEd·10⁶b·h2·fcd=108.3·10⁶800·4002·14.17=0.0598

Required reinforcement from the N-M interaction diagram of the section (symmetric reinforcement, d_1 = 45 mm, rectangular stress block, ε_cu = 3.5 ‰):

(ω used for λ_lim and K_r: ω = 0.2474 - iterated until it agrees with the result)

Mechanical reinforcement ratio (from the diagram)

ωtot=0.247

As,tot=ωtot·b·h·fcdfyd=0.247·800·400·14.17434.8=2579 mm²

Required A_s,tot = 2579 mm² (A_s,min = 1154 mm²) → adopted 14Φ16 (2815 mm², ρ = 0.8796 %)

Figure 22. Cross-section of column C7

Figure 23. Interaction diagram of column C7 with the design forces

### Column C1 - first storey (40/60 cm)

Flexibility of the restraint at the top (course formula)

k1=Icl4·Iblb=7200002904·1550090755=0.3023

Footing (minimum recommended value)

k2=0.1

Effective length in the plane of the frame (braced, EC2 5.15)

l0=0.5·l·1+k10.45+k1·1+k20.45+k2=0.5·2.9·1+0.30230.45+0.3023·1+0.10.45+0.1=1.866 m

Out of the plane of the frame

l0,out=0.7·l=0.7·2.9=2.03 m

Table 17. Design forces of column C1 (first storey)

| LC | N_Ed [kN] | M_01 [kNm] | M_02 [kNm] |
| --- | --- | --- | --- |
| LC1 | 3190 | 92.78 | -181.4 |
| LC2 | 2705 | 107.9 | -212 |
| LC3 | 2661 | 29.65 | -57.07 |

LC1:

C1 - bending in the plane of the frame: b = 400 mm, h = 600 mm

Effective length

l0=1.866 m

Radius of gyration

i=h12=60012=173.2 mm

Slenderness

λ=l0·1000i=1.866·1000173.2=10.78

Relative normal force (f_cd with α_cc = 0.85)

n=NEd·1000Ac·fcd=3190·1000240000·14.17=0.9382

EC2 5.8.3.1: A = 0.7, B = √(1+2ω), C = 1.7 − r_m

λl​im=20·A·B·Cn=20·0.7·1.209·2.2120.9382=38.64

Eccentricity from imperfections (EC2 6.1(4), 5.2(7))

ei=maxl0·1000400h3020=max1.866·10004006003020=20 mm

✔ λ ≤ λ_lim - second order effects may be ignored

M_Ed = M_02 + N_Ed·e_i

MEd=M02+NEd·ei1000=181.4+3190·201000=245.2 kNm

Relative moment

μEd=MEd·10⁶b·h2·fcd=245.2·10⁶400·6002·14.17=0.1202

Required reinforcement from the N-M interaction diagram of the section (symmetric reinforcement, d_1 = 45 mm, rectangular stress block, ε_cu = 3.5 ‰):

(ω used for λ_lim and K_r: ω = 0.2308 - iterated until it agrees with the result)

Mechanical reinforcement ratio (from the diagram)

ωtot=0.231

As,tot=ωtot·b·h·fcdfyd=0.231·400·600·14.17434.8=1805 mm²

LC2:

C1 - bending in the plane of the frame: b = 400 mm, h = 600 mm

Effective length

l0=1.866 m

Radius of gyration

i=h12=60012=173.2 mm

Slenderness

λ=l0·1000i=1.866·1000173.2=10.78

Relative normal force (f_cd with α_cc = 0.85)

n=NEd·1000Ac·fcd=2705·1000240000·14.17=0.7955

EC2 5.8.3.1: A = 0.7, B = √(1+2ω), C = 1.7 − r_m

λl​im=20·A·B·Cn=20·0.7·1.133·2.2090.7955=39.3

Eccentricity from imperfections (EC2 6.1(4), 5.2(7))

ei=maxl0·1000400h3020=max1.866·10004006003020=20 mm

✔ λ ≤ λ_lim - second order effects may be ignored

M_Ed = M_02 + N_Ed·e_i

MEd=M02+NEd·ei1000=212+2705·201000=266.1 kNm

Relative moment

μEd=MEd·10⁶b·h2·fcd=266.1·10⁶400·6002·14.17=0.1304

Required reinforcement from the N-M interaction diagram of the section (symmetric reinforcement, d_1 = 45 mm, rectangular stress block, ε_cu = 3.5 ‰):

(ω used for λ_lim and K_r: ω = 0.1421 - iterated until it agrees with the result)

Mechanical reinforcement ratio (from the diagram)

ωtot=0.142

As,tot=ωtot·b·h·fcdfyd=0.142·400·600·14.17434.8=1111 mm²

C1 - bending out of the plane of the frame (N_max): b = 600 mm, h = 400 mm

Effective length

l0=2.03 m

Radius of gyration

i=h12=40012=115.5 mm

Slenderness

λ=l0·1000i=2.03·1000115.5=17.58

Relative normal force (f_cd with α_cc = 0.85)

n=NEd·1000Ac·fcd=3190·1000240000·14.17=0.9382

EC2 5.8.3.1: A = 0.7, B = √(1+2ω), C = 1.7 − r_m

λl​im=20·A·B·Cn=20·0.7·1.06·0.70.9382=10.72

Eccentricity from imperfections (EC2 6.1(4), 5.2(7))

ei=maxl0·1000400h3020=max2.03·10004004003020=20 mm

➜ λ > λ_lim - second order effects are taken into account (nominal curvature method, EC2 5.8.8)

K_r = (n_u − n)/(n_u − n_bal) ≤ 1

Kr=0.1863

K_φ = 1 + β·φ_ef, φ_ef = 2

Kφ=1.716

1/r0=fydEs·0.45·d=434.8200000·0.45·355=0.0000136 1/mm

1/r=Kr·Kφ·1/r0=0.1863·1.716·0.0000136=0.0000043 1/mm

Second order eccentricity

e2=1/r·l0·1000210=0.0000043·2.03·1000210=1.793 mm

M_Ed = max(M_0e; M_02) + N_Ed·e_i + N_Ed·e_2

MEd=maxM0e+NEd·ei1000M02+NEd·ei1000+NEd·e21000

=max0+3190·2010000+3190·201000+3190·1.7931000=69.51 kNm

Relative moment

μEd=MEd·10⁶b·h2·fcd=69.51·10⁶600·4002·14.17=0.0511

Required reinforcement from the N-M interaction diagram of the section (symmetric reinforcement, d_1 = 45 mm, rectangular stress block, ε_cu = 3.5 ‰):

(ω used for λ_lim and K_r: ω = 0.0614 - iterated until it agrees with the result)

Mechanical reinforcement ratio (from the diagram)

ωtot=0.054

As,tot=ωtot·b·h·fcdfyd=0.054·600·400·14.17434.8=423.4 mm²

Required A_s,tot = 1805 mm² (A_s,min = 733.6 mm²) → adopted 6Φ20 (1885 mm², ρ = 0.7854 %)

Figure 24. Cross-section of column C1

Figure 25. Interaction diagram of column C1 with the design forces

Links in the columns: Φ8 (Φ10 for Φ25 bars), s ≤ min(20·Φ; b; 400 mm), 0.6·s in the critical regions at the beams and at laps.

# XI. Design of reinforced concrete pad footing for single columns

Pad footing under column C7 (40/80 cm), frame on axis 2.

## 1. Actions

Design normal force at the base (LC1, frame X)

NEd=5016 kN

Corresponding bending moment

MEd=0.0000000000102 kNm

Characteristic normal force (G_k + Q_k) - SLS

Nk=3557 kN

Mk=0.00000000000728 kNm

Weight of the footing and the backfill (≈ 15 %)

Gf,k=0.15·Nk=0.15·3557=533.5 kN

Nf,k=Nk+Gf,k=3557+533.5=4090 kN

## 2. Plan size of the footing (SLS)

Characteristic soil pressure (assignment)

R0=230 kN/m²

Af,req=Nf,kR0=4090230=17.78 m²

Equal overhangs on all sides

lc,req=0.5·Af,req−hc+bc2=0.5·17.78−0.8+0.42=1.809 m

Adopted overhang (rounded up to 5 cm)

lc=1.85 m

Lf=2·lc+hc=2·1.85+0.8=4.5 m

Bf=2·lc+bc=2·1.85+0.4=4.1 m

Af=Lf·Bf=4.5·4.1=18.45 m²

σk=Nf,kAf=409018.45=221.7 kN/m²

σk=221.7 kN/m² ≤ R0=230 kN/m²

✔ The soil pressure is acceptable

Eccentricity of the load (in the plane of the frame)

e=MkNf,k=0.000000000007284090=0 m

σk,m​ax=Nf,kAf·1+6·eLf=409018.45·1+6·04.5=221.7 kN/m²

σk,m​ax=221.7 kN/m² ≤ 1.2·R0=276 kN/m²

✔ Edge pressure acceptable

Shape (≤ 1.5 recommended)

Lf/B_f=LfBf=4.54.1=1.098

## 3. Thickness and reinforcement

Design characteristics of the materials (EN 1992-1-1):

Concrete C25/30, reinforcing steel B500.

Characteristic cylinder strength of concrete

fck=25 N/mm²

Design compressive strength

fcd=αcc·fckγc=1·251.5=16.67 N/mm²

Mean tensile strength

fctm=2.56 N/mm²

Characteristic yield strength of steel

fyk=500 N/mm²

Design yield strength of steel

fyd=fykγs=5001.15=434.8 N/mm²

Design soil pressure (self-weight not included)

σgr=NEdAf=501618.45=271.9 kN/m²

Perimeter of the column

u0=2·hc+bc=2·800+400=2400 mm

VEd,red=NEd−σgr·hc·bc=5016−271.9·0.8·0.4=4929 kN

ν=0.6·1−fck250=0.6·1−25250=0.54

From v_Ed ≤ v_Rd,max = 0.4·ν·f_cd at the column face (EC2 6.4.5(3))

dreq=VEd,red·1000u0·0.4·ν·fcd=4929·10002400·0.4·0.54·16.67=570.5 mm

Adopted height (≥ d_req + c_nom + Φ, ≥ l_c/2.5 for a rigid footing; increased if needed for punching)

Hf=900 mm

d=Hf−cnom−φ=900−50−15=835 mm

### 3.1 Bending reinforcement

Moment at the face of the column (cantilever l_c)

MEd,f=σgr·lc22=271.9·1.8522=465.3 kNm/m

Both directions (per metre)

Relative design moment

μEd=MEd,f·10⁶b·d2·fcd=465.3·10⁶1000·8352·16.67=0.04

μEd=0.04 ≤ μl​im=0.295

✔ Single reinforcement is sufficient

Lever arm ratio z/d

ζ=0.5·1+1−2·μEd=0.5·1+1−2·0.04=0.9796

Required reinforcement

As,req=MEd,f·10⁶ζ·d·fyd=465.3·10⁶0.9796·835·434.8=1308 mm²/m

Minimum reinforcement (EC2 9.2.1.1)

As,m​in=max0.26·fctmfyk·b·d0.0013·b·d=max0.26·2.56500·1000·8350.0013·1000·835=1114 mm²/m

Provided reinforcement: Φ16/15 cm  →  A_s,prov = 1340 mm²/m

Required reinforcement ratio

ρ=Asb·d=13081000·835=0.00157

Provided reinforcement ratio

ρprov=As,provb·d=13401000·835=0.00161

### 3.2 Punching shear (EC2 6.4.4(2))

The control perimeter is searched between a = 0.2·d and a = 2·d; the soil pressure inside the perimeter reduces the punching force.

Critical control perimeter (largest v_Ed/v_Rd)

acrit=709.8 mm

u=2·hc+bc+2·π·acrit=2·800+400+2·3.142·709.8=6859 mm

Ain=hc·bc+2·acrit·hc+bc+π·acrit210⁶=800·400+2·709.8·800+400+3.142·709.8210⁶=3.606 m²

VEd,red=NEd−σgr·Ain=5016−271.9·3.606=4036 kN

vEd=VEd,red·1000u·d=4036·10006859·835=0.7046 N/mm²

k=min1+200d2=min1+2008352=1.489

v_Rd = C_Rd,c·k·(100·ρ·f_ck)^(1/3)·2d/a ≥ v_min·2d/a

vRd=maxCRd,c·k·100·ρl·fck130.035·k32·fck·2·dacrit

=max0.12·1.489·100·0.00161·25130.035·1.48932·25·2·835709.8=0.7485 N/mm²

vEd=0.7046 N/mm² ≤ vRd=0.7485 N/mm²

✔ No punching shear reinforcement is required

Figure 26. Pad footing - plan and section

# XII. Drawings and bar schedules

The following drawings were produced from the results of the calculations (DXF, scale 1:50, drawing unit = 1 cm):

Formwork_Dionisios_Papadimitriou.dxf

Slab_reinforcement_Dionisios_Papadimitriou.dxf

The slabs of the hall are reinforced with the welded wire meshes adopted in section II (part 8); the bar diameters and spacings of the other slabs are the ones adopted in sections I, III, IV and VII. Distribution bars, top bars at the end supports and along the edges parallel to the span and the assembly bars follow the detailing rules given there (EC2 9.3).

Slabs 1-6: welded wire meshes (R-type, main bars along the span, laps of the transverse bars 150 mm). Top meshes on spacers.

Slabs 10 and 18 are reinforced like slabs 19-21 (two-way slabs, smaller spans).

Table 18. Bar schedule of the slabs (BR/DR bottom, TR/AR top)

| Pos. | Mark | Φ | Shape [cm] | s [cm] | n | Total [m] | Location |
| --- | --- | --- | --- | --- | --- | --- | --- |
| N1 | BR | Φ8 | 264 | 20 | 168 | 443.5 | slabs 11-16, end span; slabs 11-16, interior span |
| N2 | DR | Φ6 | 600 | 17.5 | 72 | 432 | slabs 11-16, distribution |
| N3 | BR | Φ10 | 370 | 20 | 94 | 347.8 | slab 17 |
| N4 | DR | Φ6 | 655 | 17.5 | 38 | 248.9 | slab 17, distribution |
| N5 | DR | Φ6 | 660 | 17.5 | 19 | 125.4 | slab 17, distribution |
| N6 | BR | Φ8 | 650 | 17.5 | 64 | 416 | slab 19, direction x; slab 21, direction x |
| N7 | BR | Φ8 | 600 | 16 | 101 | 606 | slab 19, direction y; slab 21, direction y; slab 10, direction y |
| N8 | BR | Φ8 | 650 | 20 | 28 | 182 | slab 20, direction x |
| N9 | BR | Φ8 | 600 | 20 | 30 | 180 | slab 20, direction y |
| N10 | BR | Φ8 | 445 | 17.5 | 32 | 142.4 | slab 10, direction x |
| N11 | BR | Φ8 | 405 | 17.5 | 37 | 149.8 | slab 18, direction x |
| N12 | BR | Φ8 | 691 | 16 | 22 | 152 | slab 18, direction y |
| N13 | TR | Φ8 | 7+165+7 | 20 | 140 | 250.6 | slabs 11-16, over the beams |
| N14 | AR | Φ6 | 600 | 25 | 89 | 534 | slabs 11-16, under TR; slabs 19-21, under TR; slab 10, under TR |
| N15 | TR | Φ8 | 15+67+7 | 20 | 188 | 167.3 | slabs 11-16, edges 5' / 7; slabs 11-16, edges on axes D, E |
| N16 | AR | Φ6 | 264 | 25 | 36 | 95.04 | slabs 11-16, edges on axes D, E |
| N17 | TR | Φ8 | 15+87+9 | 20 | 220 | 244.2 | slab 17, edges B / D; slab 17, edges 4 / 7 |
| N18 | AR | Φ6 | 655 | 25 | 16 | 104.8 | slab 17, under TR |
| N19 | AR | Φ6 | 660 | 25 | 8 | 52.8 | slab 17, under TR |
| N20 | AR | Φ6 | 370 | 25 | 8 | 29.6 | slab 17, under TR |
| N21 | TR | Φ12 | 10+335+10 | 17.5 | 64 | 227.2 | slabs 19-21, over axes 5, 6 |
| N22 | TR | Φ8 | 15+147+10 | 20 | 56 | 96.32 | slabs 19, 21, edges 4 / 7 |
| N23 | TR | Φ8 | 15+137+10 | 20 | 160 | 259.2 | slabs 19-21, edge B; slab 20, edge A; slab 10, edges |
| N24 | AR | Φ6 | 650 | 25 | 24 | 156 | slabs 19-21, under TR |
| N25 | TR | Φ8 | 15+102+10 | 20 | 56 | 71.12 | slab 10, edges |
| N26 | AR | Φ6 | 445 | 25 | 12 | 53.4 | slab 10, under TR |
| N27 | TR | Φ8 | 15+97+10 | 20 | 64 | 78.08 | slab 18, edges |
| N28 | AR | Φ6 | 691 | 25 | 8 | 55.28 | slab 18, under TR |
| N29 | TR | Φ8 | 15+152+10 | 20 | 36 | 63.72 | slab 18, edges |
| N30 | AR | Φ6 | 405 | 25 | 12 | 48.6 | slab 18, under TR |
| N31 | TR | Φ10 | 7+362+10 | 15 | 80 | 303.2 | balconies 22, 23 |
| N32 | DR | Φ6 | 600 | 17.5 | 20 | 120 | balconies 22, 23 |
| N33 | TR | Φ10 | 10+232+40 | 17.5 | 13 | 36.66 | canopy slab 7 |
| N34 | DR | Φ6 | 227 | 17.5 | 12 | 27.24 | canopy slab 7 |

## Reinforcement drawings of the elements

The reinforcement file contains also the detail drawings of the elements designed in the note (elevations, cross-sections, bars drawn separately, bar schedules): Beam B21 - simply supported beam (axis 5, A-B); Beam on axis E (3-4) - bending and torsion; Beam B5 - continuous beam over the frame beam B15; Frame on axis 2 - beam B15, columns C1, C7, C12 (first storey); Column C10 - first and second storey; Pad footing under column C7; Stairs - flight and landing (axes 3-4).

Table 19. Bar schedule of the elements

| Element | Pos. | Φ | Shape [cm] | n | Total [m] | Mass [kg] |
| --- | --- | --- | --- | --- | --- | --- |
| Beam B21 | 1 | Φ20 | 25+600+25 | 4 | 26 | 64.12 |
| Beam B21 | 2 | Φ12 | 15+600+15 | 2 | 12.6 | 11.19 |
| Beam B21 | 3 | Φ8 | 20+55+20+55+8+8 | 19 | 31.54 | 12.45 |
| Beam on axis E (3-4) | 1 | Φ12 | 15+405+15 | 5 | 21.75 | 19.31 |
| Beam on axis E (3-4) | 2 | Φ12 | 405 | 6 | 24.3 | 21.57 |
| Beam on axis E (3-4) | 3 | Φ10 | 20+45+20+45+10+10 | 36 | 54 | 33.29 |
| Beam B5 | 1 | Φ22 | 30+652 | 4 | 27.28 | 81.4 |
| Beam B5 | 2 | Φ20 | 400 | 4 | 16 | 39.46 |
| Beam B5 | 3 | Φ10 | 15+660 | 4 | 27 | 16.65 |
| Beam B5 | 4 | Φ12 | 15+148 | 4 | 6.52 | 5.789 |
| Beam B5 | 5 | Φ8 | 20+40+20+40+8+8 | 44 | 59.84 | 23.61 |
| Frame on axis 2 | 1 | Φ25 | 30+820 | 6 | 51 | 196.5 |
| Frame on axis 2 | 2 | Φ25 | 500 | 5 | 25 | 96.33 |
| Frame on axis 2 | 3 | Φ25 | 55+268 | 10 | 32.3 | 124.5 |
| Frame on axis 2 | 4 | Φ12 | 15+808 | 4 | 32.92 | 29.23 |
| Frame on axis 2 | 5 | Φ10 | 25+60+25+60+10+10 | 72 | 136.8 | 84.34 |
| Frame on axis 2 | 6 | Φ16 | 30+452 | 14 | 67.48 | 106.5 |
| Frame on axis 2 | 7 | Φ8 | 75+35+75+35+8+8 | 17 | 40.12 | 15.83 |
| Frame on axis 2 | 8 | Φ20 | 30+472 | 12 | 60.24 | 148.6 |
| Frame on axis 2 | 9 | Φ8 | 55+35+55+35+8+8 | 32 | 62.72 | 24.75 |
| Column C10 | 1 | Φ28 | 440 | 6 | 26.4 | 127.6 |
| Column C10 | 2 | Φ20 | 410 | 6 | 24.6 | 60.67 |
| Column C10 | 3 | Φ10 | 55+20+55+20+10+10 | 42 | 71.4 | 44.02 |
| Column C10 | 4 | Φ10 | 20+10+10 | 42 | 16.8 | 10.36 |
| Pad footing under column C7 | 1 | Φ16 | 20+440+20 | 27 | 129.6 | 204.6 |
| Pad footing under column C7 | 2 | Φ16 | 20+400+20 | 30 | 132 | 208.3 |
| Pad footing under column C7 | 3 | Φ16 | 30+162 | 14 | 26.88 | 42.43 |
| Pad footing under column C7 | 4 | Φ8 | 75+35+75+35+8+8 | 3 | 7.08 | 2.794 |
| Stairs | 1 | Φ10 | 330 | 12 | 39.6 | 24.41 |
| Stairs | 2 | Φ10 | 198 | 12 | 23.76 | 14.65 |
| Stairs | 3 | Φ8 | 50+119 | 9 | 15.21 | 6.002 |
| Stairs | 4 | Φ8 | 15+128 | 9 | 12.87 | 5.078 |
| Stairs | 5 | Φ8 | 165 | 25 | 41.25 | 16.28 |

Table 20. Mass of the reinforcement of the elements

| Element | Mass [kg] |
| --- | --- |
| Beam B21 | 87.75 |
| Beam on axis E (3-4) | 74.18 |
| Beam B5 | 166.9 |
| Frame on axis 2 | 826.5 |
| Column C10 | 242.7 |
| Pad footing under column C7 | 458.1 |
| Stairs | 66.42 |
| Total | 1923 |

## Slabs

Table 21. Mesh schedule of the slabs (welded wire meshes, R-type)

| Pos. | Mesh | L × B [m] | n | Area [m²] | kg/pc | Total [kg] | Location |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M1 | R221 | 2.72 × 2.11 | 12 | 68.87 | 13.53 | 162.3 | slabs 1-6, bottom, span 1 |
| M2 | R188 | 2.72 × 2.11 | 24 | 137.7 | 11.58 | 277.9 | slabs 1-6, bottom, span 2; slabs 1-6, bottom, span 3 |
| M3 | R221 | 1.65 × 2.11 | 12 | 41.78 | 8.291 | 99.49 | slabs 1-6, over support B |
| M4 | R188 | 1.65 × 2.11 | 18 | 62.67 | 7.1 | 127.8 | slabs 1-6, over support C; slabs 1-6, over support D |
| M5 | R257 | 0.72 × 2.11 | 12 | 18.23 | 4.053 | 48.64 | slabs 1-6, edges A / E |
| M6 | R257 | 0.72 × 1.21 | 24 | 20.91 | 2.411 | 57.86 | slabs 1-6, edges on axes 1, 2, 3 |
| M7 | R257 | 1.30 × 1.21 | 12 | 18.88 | 4.441 | 53.29 | slabs 1-6, edges on axes 1, 2, 3 |

Table 22. Mass of the reinforcement of the slabs

| Steel | Length [m] | kg/m | Mass [kg] |
| --- | --- | --- | --- |
| Φ6 | 2083 | 0.222 | 462.3 |
| Φ8 | 3502 | 0.3946 | 1382 |
| Φ10 | 687.7 | 0.6165 | 424 |
| Φ12 | 227.2 | 0.8878 | 201.7 |
| Meshes |  |  | 827.3 |
| Total |  |  | 3297 |

# Summary of the results

Sizes chosen automatically by the program: h_f slab I: 14; h_f slab II: 12; h_f slab III: 12; h_f slab IV: 15; h stair: 17; B21: 25/60 cm; B5: 25/45 cm; footing height: 90 cm

Table 23. Main results

| Element | Result |
| --- | --- |
| Slab 17 (corridor, simply supported) | h_f = 14 cm, 5Φ10/20 cm |
| Slabs 1-6 (hall, continuous) | h_f = 12 cm, welded meshes: span 1 R221, support B R221 |
| Slabs 11, 12, 13, 14, 15, 16 (meeting room, plastic) | h_f = 12 cm, end span 5Φ8/20 cm, support B 5Φ8/20 cm |
| Slabs 19, 20, 21 (offices, two-way) | h_f = 15 cm, x: Φ8/17.5 cm, y: Φ8/16 cm, supports: Φ12/17.5 cm |
| Balconies 22, 23 | top Φ10/15 cm |
| Stairs | h = 17 cm, 9 risers 16.11/30 cm, Φ10/14 cm |
| Beam B21 | 25/60 cm, 4Φ20, links Φ8/30 cm |
| Torsion beam (axis E, 3-4) | 25/50 cm, bottom 3Φ12, top 2Φ12, links Φ10/10 |
| Beam B5-B9 | 25/45 cm, span 2Φ22, support 4Φ20, links Φ8/25 cm |
| Column C10 | 25/60 cm, 0-0: 6Φ28; 1-1: 6Φ20 |
| Frame beam B15 | 30/65 cm: span 3Φ25, over C7 5Φ25, over C1 5Φ25, links Φ10/25 cm |
| Column C7 (1st storey) | 40/80 cm, 14Φ16 |
| Columns C1/C12 (1st storey) | 40/60 cm, 6Φ20 |
| Footing under C7 | 450 × 410 × 90 cm, Φ16/15 cm both ways |